MCS-51 Microcontroller Family Macro Assembler

AAA     SSSSSS  EEEEEEE  MM   MM        5555555   11

AA AA   SS       EE       MMM MMM        55       111

AA   AA  SS       EE       MM M MM        55        11

AA   AA   SSSSS   EEEEEEE  MM   MM  ====  555555    11

AAAAAAA       SS  EE       MM   MM             55   11

AA   AA       SS  EE       MM   MM             55   11

AA   AA  SSSSSS   EEEEEEE  MM   MM        555555   1111

U S E R ' S    M A N U A L

Version  1.3

December 31, 2002

Copyright (c) 1994, 1996, 2002  by  W.W. Heinz

TABLE OF CONTENTS

-----------------

Foreword to Version 1.0

Foreword to Version 1.2

Foreword to Version 1.3

I.    Introduction

II.   Getting started

II.1  DOS and Windows Implementation

II.1.1  Files

II.1.2  Installation under MS-DOS or Windows

II.1.3  DOS Command Line Operation

II.1.4  DOS Environment

II.1.5  Running ASEM-51 in the Borland-IDE

II.1.6  Running ASEM-51 from Windows 3.1x

II.1.7  Running ASEM-51 from BRIEF

II.1.8  The DOS Protected-Mode Assembler ASEMX

II.1.9  The Win32 Console-Mode Assembler ASEMW

II.1.10 The HEXBIN Utility

II.2  Linux Implementation

II.2.1  Files

II.2.2  Installation under Linux

II.2.3  Linux Command Line Operation

II.2.4  Linux Environment

II.2.5  The HEXBIN Utility

II.3  The DEMO Program

III.  The ASEM-51 Assembly Language

III.1  Statements

III.2  Symbols

III.3  Constants

III.4  Expressions

III.5  The 8051 Instruction Set

III.6  Pseudo Instructions

III.7  Segment Type

III.8  Assembler Controls

III.8.1  Primary Controls

III.8.2  General Controls

III.9  Predefined Symbols

III.10 Conditional Assembly

III.10.1 General IFxx Construction

III.10.2 IFxx and ELSEIFxx Instructions

III.11 Macro Processing

III.11.1 Simple Callable Macros

III.11.2 Macro Parameters

III.11.3 Repeat Macros

III.11.4 Local Symbols

III.11.5 Macro Operators

III.11.6 Premature End of a Macro Expansion

III.11.7 Nested and Recursive Macro Calls

III.11.8 Nested Macro Definitions

III.11.9 Representation in the List File

IV.   Compatibility with the Intel Assembler

IV.1  Restrictions

IV.2  Extensions

IV.3  Further Differences

V.    List File Format

VI.   Support of 8051 Derivatives

Appendix A:  ASEM-51 Error Messages

A.1  Assembly Errors

A.2  Runtime Errors

Appendix B:  HEXBIN Error Messages

B.1  Conversion Errors

B.2  Runtime Errors

Appendix C:  Predefined Symbols

Appendix D:  Reserved Keywords

Appendix E:  Specification of the Intel-HEX Format

Appendix F:  The ASCII Character Set

Appendix G:  Literature

Appendix H:  Trademarks

Appendix I:  8051 Instructions in numerical Order

Appendix J:  8051 Instructions in lexical Order

Foreword to Version 1.0

=======================

Today microcontrollers are used in a wide range of applications from simple

consumer electronic products to complex avionic components. Thus I was not

very surprised to find an 80C31 on the videotext decoder board, I purchased

some time ago. Since it had a poor user interface and many bugs, I thought

I could do it better and so I began to look for an 8051 cross assembler.

But in contrast to the huge number of hardware components sold, the number

of people developing microcontroller software seemed to be remarkable small,

and so was the number of development tools on the market.

There was a very small number of good professional cross assemblers for $250

and up - too expensive for hobby purposes. Aside of useless demos, there were

no restricted starter kits or school versions available.

I found also a few shareware and public domain assemblers. But either they

were poor and not very reliable, or not very 8051-specific, or they used some

kind of fantasy syntax that was 100 % compatible to itself, but far away from

the Intel standard. I didn't like them all!

There seems to be a general lack of useful and affordable microcontroller

development software. This is a pity, because their universality, simple

architectures and low prices make microcontrollers so interesting especially

for hobby and education.

So I decided to write a handy 8051 cross assembler for the PC.

And here it is:             ASEM-51 V1.0

I hope it will help to discover the wonderful world of microcontrollers.

Have fun!

Deisenhofen,  July 19, 1994

W.W. Heinz

Foreword to Version 1.2

=======================

More than one year has passed, since I had released ASEM-51 V1.1 in

October 1994. Although I didn't spend all the time on ASEM-51, V1.2

comes with several extensions, bug fixes, and numerous functional or

internal improvements!

Highlights of the new version are a nearly perfectly featured list

file with cross-reference and some new printing options, a bootstrap

program for MCS-51 evaluation boards, and plenty of new *.MCU files.

For detailed information see the ASEM-51 V1.2 Release Notes.

What I have learned through the last two years is that freeware is

not free, neither for the author nor for the users.

ASEM-51 could not be made with nothing but numberless free hours,

spent on pure software development. I also had to purchase a PASCAL

development system, lots of microcontroller literature, and an 80C535

evaluation board.

The distribution of freeware seems to be a bigger problem than its

development. First of all, one has to buy a modem. After that, it

costs a lot of time, fees, trouble, and "interesting" discussions

with the particular sysops, until the stuff is posted (or not) on

several BBS and ftp sites.

To publish a program on shareware CD-ROMs, one has only to find out,

which are the most suitable. For this, it is best to buy a dozen or

two (and a CD-ROM drive), and to send the software to the publishers

of those that seem to be the most popular.

The interested users finally have to purchase modems or CD-ROM drives,

and pay the same fees, or buy the same CD-ROMs, to get the "freeware"

again from these public sources.

After all, it may be cheaper, faster, and more convenient, to simply buy

a professional software solution (if any) in the PC shop at the corner.

But it's not half the fun!

ASEM-51 V1.1 had been distributed (and mirrored) to more than 60 ftp

sites all over the world, uploaded to so many BBS, and published on

at least two shareware CD-ROMs.

But I only received mails from 9 users, a local cockroach, and an

international monster. The latter two asked me for permission, to

sell ASEM-51 for (their) profit, and failed miserably.

Most of the user mails started with "I have copied your assembler

from an ftp site, which I don't remember. It is looking fine on the

first glance! By the way, have you got a data sheet of the 80Cxyz?",

or something like that.

During all the time, I have received one single error report only.

Since it had been reported by phone, I couldn't reproduce it.

Nevertheless two serious bugs have been fixed since version 1.1, but

I have found them by myself in November 1995 both.

Sure ASEM-51 is no mainstream software, but to be honest, I am a

little disappointed of the poor user feedback!

Finally, I should thank the persons, who helped me to release ASEM-51:

Andreas Kies has tested all previous beta versions of the assembler. He

has distributed the first releases, and maintained a free ASEM-51 support

account right from the beginning.

Gabriele Novak has checked the orthography of all the documentation files.

Werner Allinger has tested the latest beta version and the bootstrap program.

Last but not least, I want to thank all interested users for their comments

and suggestions.

Deisenhofen,  January 22, 1996

W.W. Heinz

Foreword to Version 1.3

=======================

Six years have passed since I had released version 1.2.

I'm almost sure that most of my users thought that this was the last one!

It seems that I had always some spare time to support, maintain, change,

extend, improve and port ASEM-51 over and over for many years, but never

to publish an official new release.

But now, after six months of beta-testing, the time has come to unveil

ASEM-51 Version 1.3 (Final Release). The most important new features are

- new host platforms: Win32 and Linux (386)

- macro processing

- dramatically improved conditional assembly

- output in absolute OMF-51 format (with debug information)

- 37 new MCU files

- documentation in both ASCII and HTML format

For detailed information on the numerous small extensions, improvements,

and bug fixes, see the ASEM-51 V1.3 Release Notes.

Since fall 2001, ASEM-51 has an official home on the Internet, the

ASEM-51 support website:  http://plit.de/asem-51/

Credits:

Andreas Kies has tested all beta versions of the assembler. Without

his Linux-knowhow, the Linux port would have been a very hard job.

Werner Allinger has tested the latest beta version, and has provided

free webspace for the ASEM-51 website right from the beginning.

Axel Kielhorn has written the file DS5000.MCU.

Anders Sandstroem has sent me 87LPC762.MCU and 87LPC768.MCU.

Michael R. has contributed 80C32X2.MCU.

I also want to thank all the other users for their bug reports,

contributions, comments and suggestions.

Bayreuth,  December 31, 2002

W.W. Heinz

I. Introduction

===============

ASEM-51 is a two-pass macro assembler for the Intel MCS-51 family of

microcontrollers.

It is running on the PC under MS-DOS, Windows, and Linux.

The DOS real-mode assembler ASEM.EXE requires only 256 kB of free DOS memory

and MS-DOS 3.0 (or higher).

The new protected-mode assembler ASEMX.EXE requires a 286 CPU (or better),

and at least 512 kB of free XMS memory.

The new Win32 console-mode assembler ASEMW.EXE requires a 386 CPU (or better)

and Windows 9x, NT, 2000 or XP.

The new Linux assembler asem requires a 386-based Linux system.

The new HTML documentation set requires a 90 MHz Pentium (or better) and a

web browser.

The ASEM-51 assembly language is a rich subset of the Intel standard that

guarantees maximum compatibility with existing 8051 assembler sources.

ASEM-51 can generate two sorts of object files: Intel-HEX format, which is

directly accepted by most EPROM programmers, and absolute OMF-51 format,

which is required for many simulators, emulators and target debuggers.

Thus ASEM-51 is suitable for small and medium MCS-51-based microcontroller

projects in hobby, education and business. However, ASEM-51 has been designed

to process also very large programs! Its most important features are:

- fast, compact, reliable, easy to use, and well-documented

- easy installation, almost no configuration required

- command line operation, batch and networking capability

- fully year 2000 compliant

- DOS (RM and PM), Win32 and Linux binaries available

- Intel-compatible syntax

- five location counters, one for each of the MCS-51 address spaces

- assembly-time evaluation of arithmetic and logical expressions

- segment type checking for instruction operands

- automatic code optimization of generic jumps and calls

- macro processing (that _really_ works)

- nested include file processing

- nested conditional assembly

- absolute OMF-51 module output (with debug information)

- Intel-HEX file output

- hex-to-binary conversion utility

- built-in symbols for 8051 special function registers (can be disabled)

- direct support of more than seventy 8051 derivatives

- support of user-defined 8051 derivatives

- special support of the Philips 83C75x family

- 8051 register bank support

- detailed assembler listing with symbol table or cross reference

- further fancy printing facilities  ;-)

- documentation in ASCII and HTML format

- bootstrap program for testing on the MCS-51 target board

- support for easy integration into the popular Borland IDE

- limited update service by the author

The ASEM-51 software package has been developed with:

Borland-Pascal mit Objekten 7.0 (c) Borland International 1992

Delphi 2.0 Client/Server Suite (c) Borland International 1996

FreePascal 1.0.6 (c) Florian Klaempfl 2002

II. Getting started

===================

This chapter describes the ASEM-51 distributions, their installation on the

supported host platforms, and how to use them in daily work.

II.1 DOS and Windows Implementation

-----------------------------------

Until version 1.2, ASEM-51 was available in a real-mode implementation for

plain MS-DOS only. Meanwhile a DOS protected-mode version and a Win32 console

mode version have been added to the package.

In contrast to the new Linux implementation, all the DOS and Windows flavours

are functionally identical and their basic operation can therefore be

described together. Only a few minor differences and special features have

to be discussed separately.

Since it should be possible to share program sources with the Linux version,

all DOS and Windows executables are able to read ASCII files in both DOS and

UNIX format, but write ASCII files in their native (DOS) format only.

II.1.1 Files

------------

Your ASEM-51 distribution archive for DOS/Windows should contain the following

groups of files:

1.)  ASEM_51.DOC        ASEM-51 User's Manual, ASCII format

DOCS.HTM        index file of the ASEM-51 documentation, HTML format

*.HTM        further pages of the HTML documentation

*.GIF        GIF images referenced by HTML pages

*.JPG        JPEG images referenced by HTML pages

ASEM.EXE        assembler (DOS real-mode)

ASEM.PIF        ASEM program information file for Windows 3.1x

ASEM.ICO        ASEM icon file for Windows 3.1x

ASEM2MSG.EXE        ASEM-51 message filter for Borland-IDE (DOS)

ASEM2MSG.PAS        Turbo-Pascal source of ASEM2MSG.EXE

ASEMX.EXE        assembler (DOS protected-mode)

ASEMX.PIF        ASEMX program information file for Windows 3.1x

ASEMX.ICO        ASEMX icon file for Windows 3.1x

DPMI16BI.OVL        Borland's 16-bit DPMI server (for ASEMX.EXE)

RTM.EXE        Borland's 16-bit DPMI runtime manager

ASEM32.BAT        runs ASEMX with Borland's 32-bit DPMI server

ASEMW.EXE        assembler (Win32 console-mode)

HEXBIN.EXE        hex-to-binary conversion utility (DOS)

HEXBINW.EXE        hex-to-binary conversion utility (Win32)

DEMO.A51        a sample 8051 assembler program

*.MCU        processor definition files of 8051 derivatives

(for a detailed list of MCU files see chapter

"VI. Support of 8051 Derivatives")

2.)   BOOT51.DOC        BOOT-51 User's Manual, ASCII format

BOOT51.HTM        index file of the BOOT-51 documentation, HTML format

BOOT51.A51        BOOT-51 assembler source (requires ASEM-51 V1.3)

CUSTOMIZ.EXE        BOOT-51 customization utility

BOOT.BAT        batch file for application program upload

UPLOAD.BAT        called by BOOT.BAT only

COMPORT.EXE        setup utility for PC serial ports

RESET51.EXE        program to reset target system via PC ports

SLEEP.EXE        program to wait for the reset recovery time

BLINK.A51        sample test program for BOOT-51

3.)   README.1ST        quick information, ASCII format

LICENSE.DOC        ASEM-51 License Agreement, ASCII format

RELEASE.130        ASEM-51 Release Notes, ASCII format

SUPPORT.DOC        ASEM-51 Support Guide, ASCII format

INSTALL.BAT        creates a proper ASEM-51 installation under MS-DOS

KILLASEM.BAT        deletes all files of the ASEM-51 package (DOS)

The first group contains all files directly associated with the assembler.

The second group contains all files directly associated with the bootstrap

program. The third group contains general support and documentation files

that apply to the whole package.

II.1.2 Installation under MS-DOS or Windows

-------------------------------------------

In principle ASEM-51 doesn't require a fuzzy software installation or

configuration. In the simplest case you can copy all files of the package

to your working directory, and enjoy the benefits of true plug-and-play

compatibility!

On the other hand, an installation of ASEM-51 under MS-DOS is very simple:

- Create a new, empty scratch directory on your harddisk.

- Unpack your ASEM-51 distribution archive into this directory,

or copy all files of the ASEM-51 package into it.

- Make the scratch directory default, run the batch file

INSTALL.BAT provided, and follow the instructions.

If you don't like anything that is running automatically, or things are not

quite clear, ASEM-51 can also be installed manually as follows:

- Create a new directory on your harddisk, e.g. C:\ASEM51.

- Copy all files of the ASEM-51 package into this directory.

- Append it to your PATH statement in file AUTOEXEC.BAT, e.g.

PATH C:\DOS;C:\UTIL;C:\ASEM51

- If this has not already been done while unpacking the distribution

archive, create a subdirectory, e.g. C:\ASEM51\MCU, and move all

the *.MCU files provided to this subdirectory, for better survey.

- Create another subdirectory, e.g. C:\ASEM51\HTML, and move all

the *.HTM, *.GIF and *.JPG files to this subdirectory, respectively.

(To read that HTML manual, invoke your web browser and start with

file C:\ASEM51\HTML\DOCS.HTM!)

- Optionally define a DOS environment variable ASEM51INC in

AUTOEXEC.BAT, to specify a search path for include files, e.g.

SET ASEM51INC=C:\ASEM51\MCU;D:\MICROS\MCS51\INCL

- For a proper operation of the Borland 16-bit DPMI server on computers

with more than 16 MB RAM, be sure that EMM386.EXE (included in DOS 5.0

or later) is loaded, and define the environment variable DPMIMEM in

AUTOEXEC.BAT as follows:

SET DPMIMEM=MAXMEM 16383

- Reboot your PC.

The installation batch file INSTALL.BAT should work properly on MS-DOS,

Windows 3.1x, and Windows 9x systems. It has not yet been tested on other

Windows versions. Especially for the NT-based Windows versions NT 4.0,

2000 and XP, a manual installation is recommended.

II.1.3 DOS Command Line Operation

---------------------------------

ASEM-51 provides full support of command line operation and batch capability

as the best commercial development tools.     ;-)

Nevertheless, it can be integrated into foreign development environments,

if desired. The assembler is invoked by typing:

ASEM <source> [<object> [<listing>]] [<options>]

where <source> is the 8051 assembler source, <object> is the output file,

and <listing> is the assembler list file. The parameters <object> and

<listing> are optional. When omitted, the file names are derived from the

<source> file name, but with extensions HEX (or OMF) and LST. All file names

may be specified without extensions. In these cases, the assembler adds

default extensions as shown below:

file             extension

------------------------------------------------------

<source>         .A51

<object>         .HEX      (with /OMF-51 option: .OMF)

<listing>        .LST

If you want a file name to have no extension, terminate it with a '.'!

Instead of file names you may also specify device names to redirect the

output to character I/O ports. Device names may be terminated with a ':'!

It is not checked, whether the device is existing or suitable for the task.

Although it is possible to read the source file from a character device

(e.g. CON:) instead of a file, this cannot be recommended: Since ASEM-51

is a two-pass assembler, it always reads the source file twice!

ASEM recognizes the following options:

/INCLUDES:path1[;path2[; ... ;pathn]]

/DEFINE:symbol[:value[:type]]

/OMF-51

/COLUMNS

/QUIET

When the /INCLUDES option is used, the assembler searches the specified

path for include files that cannot be found in the working directory.

The path may be any number of directories separated by ';' characters.

The directories will be searched from left to right.

The path, specified with the /INCLUDES option, is searched before the

path, defined with the (optional) DOS environment variable ASEM51INC!

The /DEFINE option is useful for selecting particular program variants

from the command line that have been implemented with conditional assembly.

It allows to define a symbol with a value and a segment type in the

command line. Value and type are optional. The segment type of the symbol

defaults to NUMBER, if omitted. The symbol value defaults to 0, if omitted.

The symbol value may be any numerical constant. The symbol type must be one

of the following characters:

C   =   CODE

D   =   DATA

I   =   IDATA

X   =   XDATA

B   =   BIT

N   =   NUMBER    (default)

By default, ASEM-51 generates an object file in Intel-HEX format.

When the /OMF-51 option is specified, an absolute OMF-51 module is generated.

Options may be abbreviated as long as they remain unique!

Examples:

0.)      ASEM

When invoked without parameters, the assembler displays a help screen:

MCS-51 Family Macro Assembler ASEM-51 V1.3

usage:     ASEM <source> [<object> [<listing>]] [options]

options:        /INCLUDES:path1;path2;path3

/DEFINE:symbol[:value[:type]]

/OMF-51

/COLUMNS

/QUIET

1.)      ASEM PROGRAM

will assemble the 8051 assembly language program PROGRAM.A51 and

produce an Intel-HEX file PROGRAM.HEX and a listing PROGRAM.LST.

2.)      ASEM TARZAN.ASM JANE JUNGLE.PRN

will assemble the 8051 assembly language program TARZAN.ASM and

produce an Intel-HEX file JANE.HEX and a listing JUNGLE.PRN.

3.)      ASEM PROJECT EPROM.

will assemble the 8051 assembly language program PROJECT.A51 and

produce an Intel-HEX file EPROM and a listing PROJECT.LST.

4.)      ASEM ROVER /OMF

will assemble the 8051 assembly language program ROVER.A51 and produce

an absolute OMF-51 object module ROVER.OMF and a listing ROVER.LST.

5.)      ASEM sample COM2: NUL

will assemble the 8051 assembly language program SAMPLE.A51, send

the HEX file output to the serial interface COM2 and suppress the

list file output by sending it to the NUL device.

6.)      ASEM APPLICAT /INC:C:\ASEM51\MCU;D:\MICROS\8051\HEADERS

will assemble the program APPLICAT.A51, while all required include

files will be searched first in the default directory, then in

C:\ASEM51\MCU, and finally in D:\MICROS\8051\HEADERS.

7.)      ASEM UNIVERSL /D:Eva_Board:8000H:C

will assemble the program UNIVERSL.A51, while the CODE symbol

EVA_BOARD will be predefined with value 8000H during assembly.

When program errors are detected, they are flagged on the console. This

may look as follows:

MCS-51 Family Macro Assembler ASEM-51 V1.3

APPLICAT.A51(14): must be known on first pass

USERBITS.INC(6): attempt to divide by zero

DEFINES.INC(37): symbol not defined

APPLICAT.A51(20): symbol not defined

APPLICAT.A51(27): no END statement found

5 errors detected

Every error is flagged with the name of the source or include file, the

local line number where it was found, and the error message itself.

This output format makes it easy to integrate ASEM-51 into existing foreign

development environments or workbenches.

A perfect fit for the Turbo C++ IDE (and perhaps others) can be reached

with the /COLUMNS option. When specified, the column numbers of program

errors are output additionally after the line numbers:

MCS-51 Family Macro Assembler ASEM-51 V1.3

APPLICAT.A51(14,12): must be known on first pass

USERBITS.INC(6,27): attempt to divide by zero

DEFINES.INC(37,18): symbol not defined

APPLICAT.A51(20,18): symbol not defined

APPLICAT.A51(27,1): no END statement found

5 errors detected

If errors are detected in macro expansion lines, there is no corresponding

location in the source file. Therefore, the error is flagged with the name

of the source or include file, and the local line number from where the

macro expansion has been invoked. (For callable macros this is the line

with the macro call, and for repeat blocks this is the ENDM line.)

To give the user a hint, the macro name and expansion line (and optionally

column) number are inserted before the actual error message:

MCS-51 Family Macro Assembler ASEM-51 V1.3

UARTIO.A51(44,1): RECEIVE(3,22): segment type mismatch

UARTIO.A51(87,1): REPT(4,19): symbol not defined

UARTIO.A51(87,1): REPT(8,19): symbol not defined

UARTIO.A51(87,1): REPT(12,19): symbol not defined

4 errors detected

The expansion line number is the number of the expansion line within the

corresponding macro expansion, starting with 1. If the error occurs during

expansion of a repeat block, the keyword REPT replaces the macro name.

The /QUIET option suppresses all console output except error messages.

When terminating, ASEM-51 returns an exit code to the operating system:

situation                           ERRORLEVEL

----------------------------------------------

no errors                               0

program errors detected                 1

fatal runtime error                     2

Note: Warnings do not influence the exit code!

II.1.4 DOS Environment

----------------------

To specify a search path for include files, an optional environment variable

ASEM51INC can be defined:

SET ASEM51INC=<path>

<path> may be any number of directories separated by ';' characters.

Be sure that the whole definition doesn't contain any blanks or tabs!

If ASEM51INC is defined, the assembler searches the specified <path> for

include files that can neither be found in the working directory, nor in

the search path specified with the /INCLUDES option.

The <path> directories will be searched from left to right.

Examples:

1.)      SET ASEM51INC=C:\ASEM51\MCU;D:\MICROS\MCS51\INCL

If include files can neither be found in the working directory,

nor in the /INCLUDES path (if specified), the assembler searches

next C:\ASEM51\MCU and finally D:\MICROS\MCS51\INCL.

2.)      SET ASEM51INC=C:\ASEM51\MCU;%PATH%

If ASEM51INC is defined as above in AUTOEXEC.BAT after the PATH

statement, the assembler finally searches the directory C:\ASEM51\MCU

and then all the directories, contained in the DOS program search

path, from left to right!

The maximum length of <path> is limited to 255 characters. This cannot be

exceeded with the SET command of the DOS command interpreter COMMAND.COM,

but with third party command interpreters like 4DOS (max. 512 characters)!

Note that trailing blanks and tabs behind the names of environment variables

seem to be considered significant under MS-DOS! If one subsequently defines

SET ASEM51INC =C:\ASEM51\MCU

and          SET ASEM51INC=C:\8051\MCU

there will be two (!) entries concurrently in the DOS environment! However,

the assembler will recognize the second one only. Since DOS doesn't truncate

trailing blanks and tabs from variable names, the assembler can't do this

either! That is why you should be sure, to always define the environment

variable without blanks and tabs.

II.1.5 Running ASEM-51 in the Borland-IDE

-----------------------------------------

Turbo C++ (1.0 thru 3.0) users will appreciate the possibility to invoke

ASEM-51 as a transfer program from the Borland IDE. (DOS versions only!)

For this, the filter program ASEM2MSG for the ASEM-51 error messages

has been provided. To integrate ASEM-51 into the Borland IDE, perform

the following steps:

- Be sure that ASEM-51 has been installed properly as described before,

or that ASEM.EXE and ASEM2MSG.EXE are somewhere in your PATH.

- Start the Turbo C++ (or Borland C++) IDE for DOS.

- For Turbo C++ 1.0, first click:  Options | Full menus | ON

- Click from the menu bar:         Options | Transfer

- When the "Transfer" dialog box is active, press the Edit button.

- Now the "Modify/New Transfer Item" dialog box should be active.

Fill in the following items:

Program Title:   ASEM-~51

Program Path:    ASEM

Command Line:    $NOSWAP $SAVE CUR $CAP MSG(ASEM2MSG) $EDNAME /C

Translator:      [X]

Hot key:         Shift F8

Then press the New button.

- When returned to the "Transfer" dialog box, press the OK button.

- Click from the menu bar:         Options | Save | OK

Now it should be possible, to assemble the file in the active edit

window with ASEM-51, when pressing Shift-F8. The error messages (if any)

should appear in the "Message" window. You can browse through the errors,

and jump into the source text by simply pressing <Enter>. This even works,

if the error is not in the program itself, but in an include file!

Turbo-Pascal 7.0 users can also employ their Borland IDE for assembly.

To integrate ASEM-51 into the Turbo-Pascal IDE, perform the following

steps:

- Be sure that ASEM-51 has been installed properly as described before,

or that ASEM.EXE and ASEM2MSG.EXE are somewhere in your PATH.

- Start the Turbo-Pascal 7.0 (or Borland-Pascal 7.0) IDE for DOS.

- Click from the menu bar:         Options | Tools

- When the "Tools" dialog box is active, press the New button.

- Now the "Modify/New Tool" dialog box should be active.

Fill in the following items:

Title:           ASEM-~5~1

Program path:    ASEM

Command line:    $NOSWAP $SAVE CUR $CAP MSG(ASEM2MSG) $EDNAME

Hot keys:        Shift+F8

Then press the OK button.

- When returned to the "Tools" dialog box, press the OK button.

- Click from the menu bar:         Options | Environment | Preferences

- When the "Preferences" dialog box is active, disable the "Close on

go to source" item in the "Options" checkbox. Then press the OK button.

- Finally click from the menu bar: Options | Save

Now ASEM-51 can be invoked with Shift-F8, to assemble the program in the

active edit window, while error messages (if any) appear in the "Messages"

window.

Users of both Turbo C++ and Turbo-Pascal should prefer the Turbo C++ IDE.

In the Turbo-Pascal 7.0 IDE, the /COLUMNS (or /C) option has no effect!

Turbo-Pascal versions prior to 7.0 didn't implement the Tools menu.

Note that the transfer macro $SAVE CUR saves the contents of the active

edit window (if modified), before ASEM.EXE is invoked! If your assembler

program includes further source files (which may be currently loaded into

other edit windows), better specify $SAVE ALL. This will save the contents

of all (modified) edit windows to disk files, before invoking ASEM.EXE!

If you are not sure, specify $SAVE PROMPT. This will prompt you for every

(modified) edit window to save the contents before running ASEM.EXE. For

further information on transfer macros, refer to the Borland online help!

Important: ASEM2MSG.EXE is not suitable for Win32 versions of the Borland IDE!

II.1.6 Running ASEM-51 from Windows 3.1x

----------------------------------------

Of course ASEM and ASEMX are running fine in the Windows 3.1x DOS-Box!

But for integration into the Windows 3.1x desktop, the files ASEM.PIF

and ASEM.ICO have been provided. To insert ASEM-51 into a group of the

Program Manager, perform the following steps:

- Be sure that ASEM-51 has been installed properly for MS-DOS

as described before.

- Start Windows 3.1x and expand the Program Manager window to its

full screen size representation, if necessary.

- Focus the program group in which ASEM-51 is to be inserted,

e.g. "Applications".

- Click from the Program Manager menu bar:   File | New

- When the "New Program Object" dialog box is active, choose

the option "Program Item", and click the OK button.

- Now the "Program Item Properties" dialog box should be active.

Fill in the following items:

Description:          ASEM-51

Command Line:         ASEM.PIF

Working Directory:                  (whatever you want)

Shortcut Key:                       (whatever you want)

Run Minimized:        [ ]

Then press the [Change Icon] button.

- Now a message box appears with the error message

"There are no icons available for the specified file".

Simply press the OK button.

- The "Change Icon" dialog box should be displayed now. Fill in

File Name:      ASEM.ICO

and press the OK button. Now the ASEM-51 icon should be displayed

in the icon field. Press the OK button again.

- When returned to the "Program Item Properties" dialog box, press

the OK button.

(In national Windows versions, things may look slightly different.)

Now ASEM.EXE can be invoked by simply double-clicking the ASEM-51 icon.

After entering the program parameters in a corresponding dialog box,

ASEM is running in a DOS window, which remains open after program

termination, to let you have a look on the error messages.

In principle, the installation of the protected-mode assembler ASEMX.EXE

can also be done as described above. However, the <Description> field

should be filled with "ASEM-51 XMS", the <Command Line> should be

"ASEMX.PIF", and the icon <File Name> should be ASEMX.ICO instead.

II.1.7 Running ASEM-51 from BRIEF

---------------------------------

BRIEF 3.x users can integrate ASEM-51 into their editor by simply

defining another DOS environment variable in their AUTOEXEC.BAT with

SET BCA51="ASEM %%s"

This specifies the command for compiling files with extension *.A51.

After that, ASEM-51 can be invoked from BRIEF with Alt-F10.

II.1.8 The DOS Protected-Mode Assembler ASEMX

---------------------------------------------

In general, the proven real-mode assembler ASEM.EXE is sufficient also

for very large programs. Nevertheless, it may be running out of memory,

if a program contains a huge number of long user-defined symbols, or lots

of large macro definitions.

To close the gap, the ASEM-51 package includes the new protected-mode

assembler ASEMX.EXE. ASEMX is functionally identical to ASEM, but it can

use extended memory, to meet extreme workspace requirements.

ASEMX is accompanied by Borland's 16-bit DPMI server DPMI16BI.OVL and

runtime manager RTM.EXE. It requires a 286 CPU (or better), and at least

512 kB of free XMS memory (1 MB recommended)!

When ASEMX is invoked, DPMI16BI.OVL and RTM.EXE must be either

- in your default directory,

- where ASEMX.EXE is, or

- somewhere in your PATH

During startup, the DPMI server tries to allocate all the remaining free

XMS memory for use by ASEMX. If you don't want this, you can restrict the

amount of allocated memory with the DOS environment variable DPMIMEM:

SET DPMIMEM=MAXMEM n

will restrict the XMS memory space, used for the DPMI interface, to n kB.

Never set n to a value greater than 16383!!!

In general, the Borland DPMI interface is very reliable and does normally

not conflict with other memory managers. ASEMX will also run with other

versions of DPMI16BI.OVL and RTM.EXE provided with various Borland software

packages (except TC++ 3.0 and BC++ 3.1).

However, there is trouble ahead on systems with more than 16 MB RAM!

Without specific installation, there is a fatal tendency to crash, hang,

or even boot, whenever a DPMI program like ASEMX is invoked.

For proper operation of the DPMI interface, MS-DOS 5.0 (or later) is

required, and EMM386.EXE must be loaded!

If EMM386.EXE has been loaded with parameters (e.g. NOEMS), the Borland

16-bit DPMI server cannot handle more than 16 MB! However, without

parameters (i=nnnn, x=nnnn are o.k.) or with other DPMI servers there may

be more. In these cases, ASEMX can use up to 64 MB of extended memory!

If ASEMX is running in a system environment with an own DPMI server, e.g.

the Windows DOS-Box, RTM.EXE will detect this and use the active DPMI server

instead of DPMI16BI.OVL. In this case, the environment variable DPMIMEM has

no effect.

To restrict (or increase) the available XMS memory for the Windows 3.1x DOS

prompt, change file DOSPRMPT.PIF in your WINDOWS directory with the Windows

PIF file editor.

For further information on how to make more or less XMS memory available

to application programs in other system environments, see the corresponding

user manuals.

Another interesting alternative is the Borland 32-bit DPMI server with

virtual memory management. It cannot be provided with the ASEM-51 package

for license reasons, but is contained in Borland's Turbo-Assembler 4.0

and 5.0, Borland C++ 4.5x and 5.0x, and maybe others. It has originally been

developed for the Borland command line tools, but it also works with ASEMX.

It requires a 386 CPU (or better), and allows to extend the free physical

memory with a swap file that can be created with the program MAKESWAP.EXE.

Apart of that, the 32-bit DPMI server DPMI32VM.OVL and the runtime manager

32RTM.EXE are required.

The batch file ASEM32.BAT, provided with the ASEM-51 package, shows how to

run ASEMX with 64 MB of virtual memory, using Borland's 32-bit DPMI server.

II.1.9 The Win32 Console-Mode Assembler ASEMW

---------------------------------------------

In principle, the DOS assemblers ASEM and ASEMX are also running in the

Windows 9x/NT/2000/XP DOS-Box, but with some typical DOS-specific limitations:

file names are restricted to the 8.3 format, path strings are limited to

64 characters, the real-mode assembler cannot access more than 640 kB RAM,

and DOS support is getting worse and worse with every new Windows version.

To overcome these disadvantages, the ASEM-51 package comes with the new

Win32 console-mode assembler ASEMW.EXE. ASEMW is functionally identical

to ASEM, but it can handle long file names and benefits of the Win32

memory management, which allows to assemble astronomically large programs!

Hint:  If you love file names with blanks in the middle, you have to

enclose them in double quotes, e.g.

ASEMW "Test-Program for my 80C32 Evaluation-Board.a51"

II.1.10 The HEXBIN Utility

--------------------------

Most EPROM programmers are accepting the Intel-HEX object file format that

is output by ASEM-51. However, for dumb EPROM burners and special purposes

it might be useful to convert the HEX file to a pure binary image file.

For this the conversion utility HEXBIN is provided.

It is invoked as follows:

HEXBIN <hex> [<bin>] [/OFFSET:o] [/LENGTH:l] [/FILL:f] [/QUIET]

where <hex> is the input file in Intel-HEX format, and <bin> is the

binary output file. The parameter <bin> is optional. When omitted, the

file name is derived from the <hex> file name, but with the extension BIN.

All file names may be specified without extensions. In these cases, the

program adds default extensions as shown below:

file             extension

------------------------------

<hex>            .HEX

<bin>            .BIN

If you want a file name to have no extension, terminate it with a '.'!

Instead of file names you may also specify device names to redirect the

output to character I/O ports. Device names may be terminated with a ':'!

It is not checked, whether the device is existing or suitable for the task.

The binary file output can also be controlled with the options /OFFSET,

/FILL and /LENGTH.

Normally the first byte in the binary file is the first byte of the HEX

record with the lowest load address. If a number of dummy bytes is to be

inserted on top of the file (e.g. for alignment in an EPROM image), this

can be performed with the /OFFSET option:

/OFFSET:1000

would insert 4096 dummy bytes before the first byte of the first HEX record

loaded. The offset must always be specified as a hex number. The default

offset is 0.

Since there may be peepholes between the HEX records, a fill byte value can

be defined with the /FILL option:

/FILL:0

would fill all peepholes between the HEX records with zero bytes as well as

all the dummy bytes that might have been inserted with the /OFFSET or /LENGTH

option. The fill byte value must always be specified as a hex number.

The default fill byte is the EPROM-friendly FFH.

By default the last byte in the binary file is the last byte of the HEX

record with the highest load address. If the binary file should have a

well defined length, then a number of dummy bytes can be appended to the

file (e.g. for exactly matching an EPROM length), this can be performed

with the /LENGTH option:

/LENGTH:8000

would append as many dummy bytes behind the last byte of the file, that the

total file length becomes exactly 32768 bytes. The file length must always

be specified as a hex number.

When HEXBIN has been invoked with all the above options, it may display a

file conversion report like this:

Hex File Converter HEXBIN V2.3

offset:     1000H bytes

first address:     9000H

last address:     A255H

fill peepholes with:       00H

binary image length:     8000H bytes

The /QUIET option suppresses this console output, while error messages are

displayed regardless.

Options may be abbreviated as long as they remain unique!

Examples:

0.)      HEXBIN

When invoked without parameters, HEXBIN displays a help screen:

Hex File Converter HEXBIN V2.3

usage:       HEXBIN <hexfile> [<binary>] [options]

options:     /OFFSET:offset

/LENGTH:length

/FILL:fillbyte

/QUIET

1.)      HEXBIN PROGRAM

will convert the Intel-HEX file PROGRAM.HEX to a pure binary image file

PROGRAM.BIN.

2.)      HEXBIN TARZAN.OBJ JUNGLE/FILL:E5

will convert the Intel-HEX file TARZAN.OBJ to a binary image file

JUNGLE.BIN and fill all peepholes between the HEX file records with

the binary value E5H.

3.)      HEXBIN PROJECT EPROM. /off:8000 /length:10000 /f:0

will convert the Intel-HEX file PROJECT.HEX to a binary image file

EPROM, insert 32K dummy bytes on top of file, fill all peepholes

and the dummy bytes with nulls, and extend the file to exactly 64K.

When terminating HEXBIN returns an exit code to the operating system:

situation                           ERRORLEVEL

----------------------------------------------

no errors                               0

conversion errors detected              1

fatal runtime error                     2

There is also a Win32 console-mode version of HEXBIN: HEXBINW.EXE!

HEXBINW is functionally identical to HEXBIN, but can handle long file names.

II.2 Linux Implementation

-------------------------

Until version 1.2, ASEM-51 was available for MS-DOS only.

To get rid of the original DOS "look and feel", many interfaces to the

operating system had to be modified or rewritten, e.g. command line

processing, console I/O, file handling, UNIX environment, and memory

management. Furthermore, the general behaviour of the programs had to be

adapted to UNIX conventions.

A certain rest of DOS flavour may still be remaining though.

On the other hand, the Linux binaries are able to read ASCII files in both

DOS and UNIX format. However, ASCII files are always written in UNIX format.

All these differences make it necessary to describe the Linux implementation

in a separate section!

II.2.1 Files

------------

Your ASEM-51 distribution archive for Linux should contain the following

groups of files:

1.)  asem_51.doc        ASEM-51 User's Manual, ASCII format

docs.htm        index file of the ASEM-51 documentation, HTML format

*.htm        further pages of the HTML documentation

*.gif        GIF images referenced by HTML pages

*.jpg        JPEG images referenced by HTML pages

asem            assembler (Linux 386)

asem.1          man-page for asem

hexbin            hex-to-binary conversion utility (Linux 386)

hexbin.1          man-page for hexbin

demo.a51        a sample 8051 assembler program

*.mcu        processor definition files of 8051 derivatives

(for a detailed list of MCU files see chapter

"VI. Support of 8051 Derivatives")

2.)   boot51.doc        BOOT-51 User's Manual, ASCII format

boot51.htm        index file of the BOOT-51 documentation, HTML format

boot51.a51        BOOT-51 assembler source (for ASEM-51 V1.3 and up)

customiz          BOOT-51 customization utility (Linux 386)

customiz.1        man-page for customiz

boot              shell script for application program upload

boot.1            man-page for boot

upload            called by boot only (generic version)

upload.new        "new" upload (optimized for stty 2.0 or later)

reset51           program to reset the target system via PC ports

reset51.1         man-page for reset51

blink.a51         sample test program for BOOT-51

3.)   README.1ST        quick information, ASCII format

license.doc        ASEM-51 License Agreement, ASCII format

release.130        ASEM-51 Release Notes, ASCII format

support.doc        ASEM-51 Support Guide, ASCII format

install.sh         creates a proper ASEM-51 installation under Linux

uninst51.sh         deletes all files of the ASEM-51 package (Linux)

II.2.2 Installation under Linux

-------------------------------

ASEM-51 for Linux is available as a tar archive and an rpm package.

If you have got the rpm package, login as root and simply type

rpm -i asem51-1.3-1.i386.rpm

The rpm package has been tested on S.u.S.E.-Linux only, but should also

work on other Linux distributions that meet the FHS directory standard.

If you have got the tar archive, perform the following steps:

gzip -d asem51-1.3-ELF.tar.gz

tar xvf asem51-1.3-ELF.tar

cd asem51

sh install.sh

If you are installing ASEM-51 as root (preferred), the installation script

install.sh will install the whole package in /usr/local/share/asem-51/1.3,

and establish some symbolic links in /usr/local/bin and /usr/local/man/man1.

If you are installing ASEM-51 under another user-id, install.sh tries to

install the software in your home directory under ~/asem-51/1.3, and

establish some symbolic links in ~/bin and ~/man/man1.

For details see the messages, install.sh is displaying on the console,

and do some fine-tuning accordingly:

If you haven't installed ASEM-51 as root, it may be necessary to add

~/bin to your PATH, and ~/man to your MANPATH.

To specify a search path for the include files *.mcu provided, you can

define an optional environment variable ASEM51INC.

For this, bash, ksh, and sh users should insert the following lines into

their .profile file:

ASEM51INC=/usr/local/share/asem-51/1.3/mcu

export ASEM51INC

csh, tcsh, and zsh users should insert the following line into their

.login file respectively:

setenv ASEM51INC /usr/local/share/asem-51/1.3/mcu

If you have installed ASEM-51 in your home directory, ASEM51INC should

point to ~/asem-51/1.3/mcu of course.

To read the HTML manuals, invoke your web browser and bookmark the index page

/usr/local/share/asem-51/1.3/html/docs.htm    (installation as root)

~/asem-51/1.3/html/docs.htm                   (local installation)

Note that you cannot reset your 8051 target system with a PC printer port,

if you haven't installed ASEM-51 as root!

(For details see the BOOT-51 documentation provided.)

If you have installed ASEM-51, but you don't like it, you can easily

uninstall it. If you have installed the rpm package, simply type

rpm -e asem51

If you have installed the generic tar archive, be sure to uninstall

ASEM-51 under the same user-id you previously used for installation! Run

uninst51.sh

and that's it.

II.2.3 Linux Command Line Operation

-----------------------------------

Under Linux, the assembler is invoked by typing:

asem [<options>] <source> [<object> [<listing>]]

where <source> is the 8051 assembler source, <object> is the output

file, and <listing> is the assembler list file.

All file names that are specified explicitly, are left unchanged.

The parameters <object> and <listing> are optional. When omitted, the

<object> file name is derived from the <source> file name, but with

extension ".hex" (or ".omf"). When the <listing> file name is omitted,

it is derived from the <object> file name, but with extension ".lst":

file             extension

-----------------------------------------------

<object>         .hex    (with -o option: .omf)

<listing>        .lst

Instead of file names you may also specify device names to redirect the

output to I/O devices. Device names are assumed to start with "/dev/".

Of course no extensions will be added to device names!

It is not checked, whether the device is existing or suitable for the task.

Although it is possible to read the source file from a character device

instead of a file, this cannot be recommended: Since ASEM-51 is a two-pass

assembler, it always reads the source file twice!

The maximum length of a file parameter is limited to 255 characters!

asem recognizes the following options:

short options             |  long options

--------------------------+--------------------------------

-i path1:path2:path3      |  --includes=path1:path2:path3

-d symbol[:value[:type]]  |  --define=symbol[:value[:type]]

-o                        |  --omf-51

-c                        |  --columns

-v                        |  --verbose

The short and long options in the same row are equivalent.

Long options may be abbreviated as long as they remain unique.

All option names are case-sensitive!

When the --includes option is used, the assembler searches the specified

path for include files that cannot be found in the working directory.

The path may be any number of directories separated by ':' characters.

The directories will be searched from left to right.

The path, specified with the --includes option, is searched before the

path, defined with the (optional) environment variable ASEM51INC!

The maximum path length is limited to 255 characters.

The --define option is useful for selecting particular program variants

from the command line that have been implemented with conditional assembly.

It allows to define a symbol with a value and a segment type in the

command line. Value and type are optional. The segment type of the symbol

defaults to NUMBER, if omitted. The symbol value defaults to 0, if omitted.

The symbol value may be any numerical constant. The symbol type must be one

of the following characters:

C   =   CODE

D   =   DATA

I   =   IDATA

X   =   XDATA

B   =   BIT

N   =   NUMBER    (default)

By default, ASEM-51 generates an object file in Intel-HEX format. When

the --omf-51 option is specified, an absolute OMF-51 module is generated.

Examples:

0.)      asem

When invoked without parameters, the assembler displays a help screen:

MCS-51 Family Macro Assembler ASEM-51 V1.3

usage:     asem [options] <source> [<object> [<listing>]]

options:   -i   --includes=path1:path2:path3

-d   --define=symbol[:value[:type]]

-o   --omf-51

-c   --columns

-v   --verbose

1.)      asem program.a51

will assemble the 8051 assembly language program program.a51 and

produce an Intel-HEX file program.hex and a listing program.lst.

2.)      asem tarzan.asm jane jungle.prn

will assemble the 8051 assembly language program tarzan.asm and

produce an Intel-HEX file jane and a listing jungle.prn.

3.)      asem project eprom

will assemble the 8051 assembly language program project and

produce an Intel-HEX file eprom and a listing eprom.lst.

4.)      asem -o rover.a51

will assemble the 8051 assembly language program rover.a51 and produce

an absolute OMF-51 object module rover.omf and a listing rover.lst.

5.)      asem sample.a51 /dev/ttyS0 /dev/null

will assemble the 8051 assembly language program sample.a51, send

the HEX file output to the serial interface /dev/ttyS0 and suppress

the list file output by sending it to the /dev/null device.

6.)      asem -i /usr/local/include/asem-51:~/8051/inc app.a51

will assemble the program app.a51, while all required include

files will be searched first in the default directory, then in

/usr/local/include/asem-51, and finally in ~/8051/inc.

7.)      asem --define=Eva_Board:8000H:C universal.a51

will assemble the program universal.a51, while the CODE symbol

EVA_BOARD will be predefined with value 8000H during assembly.

When program errors are detected, corresponding error messages are output

to standard error. This may look as follows:

applicat.a51(14): must be known on first pass

userbits.inc(6): attempt to divide by zero

defines.inc(37): symbol not defined

applicat.a51(20): symbol not defined

applicat.a51(27): no END statement found

Every error is flagged with the name of the source or include file, the

local line number where it was found, and the error message itself.

This output format provides a hook to run ASEM-51 from third-party IDEs.

A perfect fit may be reached with the --columns option. When specified,

the column numbers of program errors are output additionally after the

line numbers:

applicat.a51(14,12): must be known on first pass

userbits.inc(6,27): attempt to divide by zero

defines.inc(37,18): symbol not defined

applicat.a51(20,18): symbol not defined

applicat.a51(27,1): no END statement found

If errors are detected in macro expansion lines, there is no corresponding

location in the source file. Therefore, the error is flagged with the name

of the source or include file, and the local line number from where the

macro expansion has been invoked. (For callable macros this is the line

with the macro call, and for repeat blocks this is the ENDM line.)

To give the user a hint, the macro name and expansion line (and optionally

column) number are inserted before the actual error message:

uartio.a51(44,1): RECEIVE(3,22): segment type mismatch

uartio.a51(87,1): REPT(4,19): symbol not defined

uartio.a51(87,1): REPT(8,19): symbol not defined

uartio.a51(87,1): REPT(12,19): symbol not defined

The expansion line number is the number of the expansion line within the

corresponding macro expansion, starting with 1. If the error occurs during

expansion of a repeat block, the keyword REPT replaces the macro name.

By default, ASEM-51 is totally "quiet", if no errors are detected.

If the --verbose option is specified, additional product, version, and

error summary information is written to standard output:

MCS-51 Family Macro Assembler ASEM-51 V1.3

uartio.a51(44,1): RECEIVE(3,22): segment type mismatch

uartio.a51(87,1): REPT(4,19): symbol not defined

uartio.a51(87,1): REPT(8,19): symbol not defined

uartio.a51(87,1): REPT(12,19): symbol not defined

4 errors detected

When terminating, ASEM-51 returns an exit code to the calling process:

situation                           exit code

---------------------------------------------

no errors                               0

program errors detected                 1

fatal runtime error                     2

Note: Warnings are also output on standard error,

but do not influence the exit code!

II.2.4 Linux Environment

------------------------

To specify a search path for include files, an optional environment variable

ASEM51INC can be defined:

1.) For bash, ksh, and sh:

ASEM51INC=<path>

export ASEM51INC

2.) For csh, tcsh, and zsh:

setenv ASEM51INC <path>

<path> may be any number of directories separated by ':' characters.

Be sure that the whole definition doesn't contain any blanks or tabs!

If ASEM51INC is defined, the assembler searches the specified <path> for

include files that can neither be found in the working directory, nor in

the search path specified with the --includes option.

The <path> directories will be searched from left to right.

Examples:

1.) bash:

ASEM51INC=/usr/local/include/asem-51:~/micros/mcs51/inc

export ASEM51INC

If include files can neither be found in the working directory,

nor in the --includes path (if specified), the assembler searches

next /usr/local/include/asem-51 and finally ~/micros/mcs51/inc.

2.) csh:

setenv ASEM51INC /usr/local/include/asem-51

If ASEM51INC is defined as above in .login, the assembler finally

searches the directory /usr/local/include/asem-51 for include files.

The maximum length of <path> is limited to 255 characters.

II.2.5 The HEXBIN Utility

-------------------------

Most EPROM programmers are accepting the Intel-HEX object file format that

is output by ASEM-51. However, for dumb EPROM burners and special purposes

it might be useful to convert the HEX file to a pure binary image file.

For this the conversion utility hexbin is provided.

It is invoked as follows:

hexbin [<options>] <hexfile> [<binary>]

where <hexfile> is the input file in Intel-HEX format, and <binary> is the

binary output file. All file names that are specified explicitly, are left

unchanged. The parameter <binary> is optional. When omitted, the file name

is derived from the <hexfile>, but with the extension ".bin".

The maximum length of a file parameter is limited to 255 characters!

Instead of file names you may also specify device names to redirect the input

or output to I/O devices. Device names are assumed to start with "/dev/".

Of course no extensions will be added to device names!

It is not checked, whether the device is existing or suitable for the task.

hexbin recognizes the following options:

short options  |  long options

---------------+---------------------

-o <offset>    |  --offset=<offset>

-l <length>    |  --length=<length>

-f <fillbyte>  |  --fill=<fillbyte>

-v             |  --verbose

The short and long options in the same row are equivalent.

Long options may be abbreviated as long as they remain unique.

All option names are case-sensitive!

The binary file output can be controlled with the options --offset,

--fill and --length.

Normally the first byte in the binary file is the first byte of the HEX

record with the lowest load address. If a number of dummy bytes is to be

inserted on top of the file (e.g. for alignment in an EPROM image), this

can be performed with the --offset option:

--offset=1000

would insert 4096 dummy bytes before the first byte of the first HEX record

loaded. The offset must always be specified as a hex number. The default

offset is 0.

Since there may be peepholes between the HEX records, a fill byte value can

be defined with the --fill option:

--fill=0

would fill all peepholes between the HEX records with zero bytes as well

as all the dummy bytes that might have been inserted with the --offset or

--length option. The fill byte value must always be specified as a hex

number. The default fill byte is the EPROM-friendly FFH.

By default the last byte in the binary file is the last byte of the HEX

record with the highest load address. If the binary file should have a

well defined length, then a number of dummy bytes can be appended to the

file (e.g. for exactly matching an EPROM length), this can be performed

with the --length option:

--length=8000

would append as many dummy bytes behind the last byte of the file, that the

total file length becomes exactly 32768 bytes. The file length must always

be specified as a hex number.

By default, hexbin is totally "quiet", if no errors are detected.

If the --verbose option is specified, additional product and version

information, and a file conversion report is written to standard output:

Hex File Converter HEXBIN V2.3

offset:      FF0H bytes

first address:     7FF0H

last address:     8255H

fill peepholes with:       A5H

binary image length:     2000H bytes

Examples:

0.)      hexbin

When invoked without parameters, hexbin displays a help screen:

Hex File Converter HEXBIN V2.3

usage:      hexbin [options] <hexfile> [<binary>]

options:   -o   --offset=<offset>

-l   --length=<length>

-f   --fill=<fillbyte>

-v   --verbose

1.)      hexbin program.hex

will convert the Intel-HEX file program.hex to a pure binary image file

program.bin.

2.)      hexbin -f E5 tarzan.obj jungle.bin

will convert the Intel-HEX file tarzan.obj to a binary image file

jungle.bin and fill all peepholes between the HEX file records with

the binary value E5H.

3.)      hexbin --off=8000 -l10000 --fill=0 project.hex eprom

will convert the Intel-HEX file project.hex to a binary image file

eprom, insert 32K dummy bytes on top of file, fill all peepholes

and the dummy bytes with nulls, and extend the file to exactly 64K.

When terminating hexbin returns an exit code to the calling process:

situation                           exit code

---------------------------------------------

no errors                               0

conversion errors detected              1

fatal runtime error                     2

II.3 The DEMO Program

---------------------

For getting started with a new assembler, it is always helpful to have

a program that can be assembled with it. For this purpose, the 8051

assembler program DEMO.A51 is provided, which can be used for a first

test of the ASEM-51 installation. For this, you should either have installed

ASEM-51 as described above, or keep all files of the ASEM-51 package directly

in your working directory!

Under MS-DOS or in a Windows DOS-Box simply type

ASEM DEMO

HEXBIN DEMO

at the DOS prompt. ASEM and HEXBIN should finish without errors and you

should have the following new files on your disk:

DEMO.HEX          Intel-HEX file

DEMO.LST          assembler list file of DEMO.A51

DEMO.BIN          binary image file of DEMO.HEX

Under Linux type

asem demo.a51

hexbin demo.hex

Again asem and hexbin should finish without errors and you should have

the following new files on your disk:

demo.hex          Intel-HEX file

demo.lst          assembler list file of demo.a51

demo.bin          binary image file of demo.hex

If something goes wrong, either ASEM-51 is not properly installed, there

may be files missing in your distribution, or the assembler simply cannot

find the include file 8052.mcu!

demo.a51 may also serve as a sample assembler program that includes examples

for (nearly) all machine instructions, pseudo instructions, assembler

controls, and meta instructions that have been implemented in ASEM-51.

Whenever in doubt how to use a particular command, demo.a51 may be a

valuable help.

Unlike other assemblers, the ASEM-51 list file is no alibi feature!

It is really instructive to compare the original source to the generated

code in the listing.

III. The ASEM-51 Assembly Language

==================================

The user should be familiar with 8051 microcontrollers and assembly

language programming. This manual will not explain the architecture of

the MCS-51 microcontroller family nor will it discuss the basic concepts

of assembly language programming. It only describes the general syntax

of assembler statements and the assembler instructions that have been

implemented in ASEM-51.

III.1 Statements

----------------

Source files consist of a sequence of statements of one of the forms:

[symbol:]  [instruction [arguments]]     [;comment]

symbol     instruction  argument        [;comment]

$control   [(argument)]                  [;comment]

Everything that is written in brackets is optional.

The maximum length of source code lines is 255 characters.

Everything from the ';' character to the end of line is assumed to be

commentary. Blank lines are considered to be commentary, too.

The lexical elements of a statement may be separated by blanks and tabs.

Aside of character string constants, upper and lower case letters are

equivalent.

Examples:     HERE:   MOV A,#0FFH     ;define label HERE and load A with FFH

YEAR EQU 2002   ;define symbol for current year

$INCLUDE (80C517.MCU)   ;include SAB80C517 register definitions

III.2 Symbols

-------------

Symbols are user-defined names for addresses, numbers or macros.

Their maximum significant length is 31 characters. They can be even

longer, but everything behind the first 31 characters is ignored.

Symbols may consist of letters, digits, '_' and '?' characters.

A symbol name must not start with a digit!

Upper and lower case letters are considered to be equivalent.

Note: Assembly language keywords must not be redefined as user symbols!

Example:     Is_this_really_a_SYMBOL_?       is a legal symbol name!

III.3 Constants

---------------

Numeric constants consist of a sequence of digits, followed by a radix

specifier. The first character must always be a decimal digit.

The legal digits and radix specifiers are:

constant     digits        radix

------------------------------------

binary       0 ... 1       B

octal        0 ... 7       Q or O

decimal      0 ... 9       D or none

hex          0 ... F       H

Thus, for example, the following constants are equivalent:

1111111B      binary

177Q      octal

177o      octal

127      decimal

127d      decimal

07FH      hex

Character constants may be used wherever a numeric value is allowed.

A character constant consists of one or two printing characters enclosed

in single or double quotes. The quote character itself can be represented

by two subsequent quotes. For example:

'X'       8 bit constant:       58H

"a@"      16 bit constant:     6140H

''''       8 bit constant:       27H

In DB statements, character constants may have any length.

In this case, we call it a character string. For example:

'This is only text!'

III.4 Expressions

-----------------

Arithmetic expressions are composed of operands, operators and parentheses.

Operands may be user-defined symbols, constants or special assembler symbols.

All operands are treated as unsigned 16-bit numbers.

Special assembler symbols, that can be used as operands are:

AR0, ... , AR7     direct addresses of registers R0 thru R7

$                  the location counter of the currently active segment

(start address of the current assembler statement)

The following operators are implemented:

Unary operators:   +          identity:             +x = x

-          two's complement:     -x = 0-x

NOT        one's complement:  NOT x = FFFFH-x

HIGH       high order byte

LOW        low order byte

Binary operators:  +          unsigned addition

-          unsigned subtraction

*          unsigned multiplication

/          unsigned division

MOD        unsigned remainder

SHL        logical shift left

SHR        logical shift right

AND        logical and

OR         logical or

XOR        exclusive or

.          bit operator used for bit-adressable locations

EQ  or =   equal to                ----.

NE  or <>  not equal to                |  results are:

LT  or <   less than                   |

LE  or <=  less or equal than          |      0 if FALSE

GT  or >   greater than                |  FFFFH if TRUE

GE  or >=  greater or equal than   ____|

Operators that are no special characters but keywords as SHR or AND must

be separated from their operands by at least one blank or tab.

In general expressions are evaluated from left to right according to

operator precedence, which may be overridden by parentheses.

Parentheses may be nested to any level.

Expressions always evaluate to unsigned 16-bit numbers, while overflows

are ignored. When an expression result is to be assigned to an 8-bit

quantity, the high byte must be either 00 or FF.

Operator precedence:

-----------------------------------------------------------------

( )                                                   ^   highest

+ - NOT HIGH LOW                       (unary)        |

.                                                     |

* / MOD                                               |

SHL SHR                                               |

+ -                                    (binary)       |

EQ = NE <> LT < LE <= GT > GE >=                      |

AND                                                   |

OR XOR                                                v   lowest

-----------------------------------------------------------------

Example:  The expression  P1.((87+3)/10 AND -1 SHR 0DH)  will evaluate to 91H.

III.5 The 8051 Instruction Set

------------------------------

ASEM-51 implements all 8051 machine instructions including generic jumps

and calls. The assembler implements two instructions

JMP  <address>

CALL <address>

that do not represent a specific opcode: generic jump and call.

These instructions will always evaluate to a jump or call, not necessarily

the shortest, that will reach the specified address.

JMP may assemble to SJMP, AJMP or LJMP, while CALL can only evaluate to

ACALL or LCALL. Note that the assembler decision may not be optimal. For

code addresses that are forward references, the assembler always generates

LJMP or LCALL respectively. However, for backward references this is a

powerful tool to reduce code size without extra trouble.

With the $PHILIPS control, ASEM-51 can be switched to the reduced instruction

set of the Philips 83C75x family of microcontrollers. This disables the LJMP,

LCALL, and MOVX instructions as well as the XDATA and XSEG pseudo instructions,

and generic jumps and calls will always assemble to absolute addressing.

The rest of the 8051 instruction mnemonics is listed in Appendix D.

Appendices I and J are containing tables of all 8051 instructions with

their opcodes, mnemonics, arguments, lengths, affected flags and durations.

The comprehensive example program DEMO.A51 provided shows all the 8051

instructions in a syntactical context.

For detailed information on the Intel MCS-51 architecture and instruction

set refer to the HTML documentation file MCS51MAN.HTM provided.

(Requires a web-browser and full Internet access!)

All MCS-51 instruction mnemonics are copyright (c) by Intel Corporation!

III.6 Pseudo Instructions

-------------------------

In the subsequent paragraphs, all ASEM-51 pseudo instructions are described.

Lexical symbols are written in lower case letters, while assembler keywords

are written in upper case.

Instruction arguments are represented by <arg>, <arg1> or something like

that. Numeric expressions are represented by <expr>, <expr1> and so on.

Syntax elements enclosed in brackets are optional.

The ellipsis "..." means always "a list with any number of elements".

DB <arg1> [,<arg2> [,<arg3> ... ]]              define bytes

The DB instruction reserves and initializes a number of bytes with

the values defined by the arguments. The arguments may either be

expressions (which must evaluate to 8-bit values) or character

strings of any length. DB is only allowed in the CODE segment!

Example:   DB 19,'January',98,(3*7+12)/11

DW <expr1> [,<expr2> [,<expr3> ... ]]           define words

The DW instruction reserves and initializes a number of words with

the values defined by the arguments. Every argument may be an

arbitrary expression and requires two bytes of space.

DW is only allowed in the CODE segment!

Example:   DW 0,0C800H,1999,4711

DS <expr>                                       define space

Reserves a number of uninitialized bytes in the current segment.

The value of <expr> must be known on pass 1!

DS is allowed in every segment, except in the BIT segment!

Example:   DS 200H

DBIT <expr>                                     define bits

Reserves a number of uninitialized bits.

The value of <expr> must be known on pass 1!

DBIT is only allowed in the BIT segment!

Example:   DBIT 16

NAME <symbol>                                   define module name

Defines a module name for the OMF-51 object file. If no module name

is defined, the module name is derived from the source file name.

When generating Intel-HEX file output, the NAME instruction has no

effect. The module name must be a legal assembler symbol.

Only one NAME instruction is allowed within the program. The symbol

however, may be redefined in the subsequent program.

Example:   NAME My_1st_Program

ORG <expr>                                      origin of segment location

Sets the location counter of the current segment to the value <expr>.

The value of <expr> must be known on pass 1!

It must be greater or equal to the segment base address.

The default value of all location counters at program start is 0.

Example:   ORG 08000H

USING <expr>                                    using register bank

Sets the register bank used to <expr>, which must be in the range

of 0...3. The USING instruction only affects the values of the

special assembler symbols AR0, ... , AR7 representing the direct

addresses of registers R0, ... , R7 in the current register bank.

The value of <expr> must be known on pass 1!

The default value for the register bank is 0.

Example:   USING 1

END                                             end of program

This must be the last statement in the source file. After the END

statement only commentary and blank lines are allowed!

Example:   END       ;end of program

<symbol> EQU <expr>                             define numeric constant

<symbol> EQU <reg>                              define invariant register

<symbol> SET <expr>                             define numeric variable

<symbol> SET <reg>                              define variable register

The EQU instruction defines a symbol for a numeric constant or a

register. If a numeric expression <expr> is assigned to the symbol,

it will be of the type NUMBER. If a register <reg> is assigned to

the symbol, it will be of the type REGISTER. <reg> may be one of the

special assembler symbols A, R0, R1, R2, R3, R3, R4, R5, R6, or R7.

A symbol once defined with EQU can never be changed!

The SET instruction is working quite similar to EQU. However, symbols

defined with SET can be redefined with subsequent SET instructions!

The values of <expr> and <reg> must be known on pass 1!

A symbol that has been SET, cannot be redefined with EQU!

A symbol that has been EQU'd cannot be reSET!

On pass 2, forward references to a SET symbol always evaluate

to the last value, the symbol has been SET to on pass 1.

Register symbols can be used as instruction operands within the

whole program instead of the corresponding registers.

Forward references to register symbols are not allowed!

Examples:   MAXMONTH  EQU 12

OCTOBER   EQU MAXMONTH-2

COUNTREG  EQU R5

CHAPTER   SET  1

CHAPTER   SET  CHAPTER+1

CHAPTER   SET  A

<symbol> CODE  <expr>                           define ROM address

<symbol> DATA  <expr>                           define direct RAM address

<symbol> IDATA <expr>                           define indirect RAM address

<symbol> BIT   <expr>                           define bit address

<symbol> XDATA <expr>                           define external RAM address

These instructions define symbolic addresses for the five 8051

memory segments (address spaces). For DATA, IDATA and BIT type

symbols, the value of <expr> must not exceed 0FFH!

The value of <expr> must be known on pass 1!

Once defined with one of the above instructions, the symbols cannot

be redefined.

Examples:   EPROM    CODE  08000H

STACK    DATA       7

V24BUF   IDATA   080H

REDLED   BIT     P1.5

SAMPLER  XDATA  0100H

CSEG [AT <expr>]                         switch to CODE  segment [at address]

DSEG [AT <expr>]                         switch to DATA  segment [at address]

ISEG [AT <expr>]                         switch to IDATA segment [at address]

BSEG [AT <expr>]                         switch to BIT   segment [at address]

XSEG [AT <expr>]                         switch to XDATA segment [at address]

These instructions switch to one of the five 8051 address spaces.

If a segment base address is specified with "AT <expr>", a new

absolute segment is started, and the location counter is set to

<expr>. If "AT <expr>" is omitted, the location counter keeps the

previous value of the particular segment.

The value of <expr> must be known on pass 1!

At program start, the default segment is CODE and the base addresses

and location counters of all segments are set to zero.

Examples:   DSEG           ;switch to previous DATA segment

CSEG AT 8000h  ;start a new CODE segment at address 8000H

XSEG at 0      ;start a new XDATA segment at address 0

III.7 Segment Type

------------------

Every assembly time expression is assigned a segment type, depending on

its operands and operators. The segment type indicates the address space,

the expression result might belong to, if it were used as an address.

There are six possible segment types:

CODE

DATA

IDATA

XDATA

BIT

NUMBER    (typeless)

Most expression results have the segment type NUMBER. That means they are

assumed to be typeless. However, in some cases it may be useful to assign

a particular segment type!

The following six rules apply when the segment type is evaluated:

1. Numerical constants are always typeless.

Consequently their segment type is NUMBER.

2. Symbols are assigned a segment type during definition. Symbols

that are defined with EQU or SET have no segment type.

Labels get the segment type of the currently active segment.

3. The result of a unary operation (+, -, NOT, HIGH, LOW) will have

the segment type of its operand.

4. The results of all binary operations (except "+", "-" and ".") will

have no segment type.

5. If only one operand in a binary "+" or "-" operation has a segment

type, then the result will have that segment type, too. In all other

cases, the result will have no segment type.

6. The result of the bit operation "." will always have the segment

type BIT.

Examples:

---------       The following symbols have been defined in a program:

OFFSET  EQU   16

START   CODE  30H

DOIT    CODE  0100H

REDLED  BIT   P1.3

VARIAB4 DATA  20H

PORT    DATA  0C8H

RELAY   EQU   5

1.)  The expression   START+OFFSET+3   will have the segment type CODE.

2.)  The expression   START+DOIT       will be typeless.

3.)  The expression   DOIT-REDLED      will be typeless.

4.)  The expression   2*VARIAB4        will be typeless.

5.)  The expression   PORT.RELAY       will have the segment type BIT.

The segment type is checked, when expressions appear as addresses. If the

expression result is not typeless and does not have the segment type of the

corresponding segment, the instruction is flagged with an error message.

The only exceptions are the segment types DATA and IDATA, which are assumed

to be compatible in the address range of 0 to 7FH. Since ASEM-51 does only

support absolute segments, those addresses are really always pointing to the

same physical location in the internal memory.

Example:

--------

Line  I  Addr  Code            Source

1:          N        30             DSEG AT 030H     ;internal RAM

2:      30  N        01     COUNT:  DS 1             ;counter variable

3:

4:                                  CSEG             ;ROM

5:    0000  C2 30           START:  CLR COUNT

^

@@@@@ segment type mismatch @@@@@

The CLR instruction is flagged with the error message "segment type mismatch"

in the assembler list file, because only a BIT type address is allowed here.

However, COUNT is a label with the segment type DATA!

III.8 Assembler Controls

------------------------

ASEM-51 implements a number of assembler controls that influence the

assembly process and list file generation. There are two groups of

controls: primary and general controls.

Primary controls can only be used at the beginning of the program and

remain in effect throughout the assembly. They may be preceded only

by control statements, blank and commentary lines. If the same primary

control is used multiple times with different parameters, the last one

counts.

General controls may be used everywhere in the program. They perform a

single action, or remain in effect until they are cancelled or changed by

a subsequent control statement.

A control statement starts always with a '$' character, followed by one or

more assembler controls.

Assembler controls may have a number or string type operand, which must

always be enclosed in parentheses.

Number type operands are arithmetic expressions that must be known on pass 1.

String type operands are character strings which are enclosed in parentheses

instead of quotes. In analogy to quoted strings, no control characters

(including tabs) are allowed within these strings! The string delimiter ')'

can be represented by two subsequent ')' characters.

If a control statement changes the listing mode, the control statement itself

is always listed in the previous listing mode!

The following table lists all the implemented controls and their abbreviations:

Control        Type  Default     Abbreviation            Meaning

-------------------------------------------------------------------------------

$COND            G   $COND       ---      list full IFxx .. ENDIF constructions

$NOCOND          G               ---      don't list lines in false branches

$CONDONLY        G               ---      list assembled lines only

-------------------------------------------------------------------------------

$DATE(string)    P      ''       $DA      inserts date string into page header

-------------------------------------------------------------------------------

$DEBUG           P   $NODEBUG    $DB      include debug information into object

$NODEBUG         P               $NODB    don't include debug information

-------------------------------------------------------------------------------

$EJECT           G               $EJ      start a new page in list file

-------------------------------------------------------------------------------

$ERROR(string)   G               ---      force a user-defined error

$WARNING(string) G               ---      output a warning message to console

-------------------------------------------------------------------------------

$GEN             G   $GEN        $GE      list macro calls and expansion lines

$NOGEN           G               $NOGE    list macro calls only

$GENONLY         G               $GO      list expansion lines only

-------------------------------------------------------------------------------

$INCLUDE(file)   G               $IC      include a source file

-------------------------------------------------------------------------------

$LIST            G   $LIST       $LI      list subsequent source lines

$NOLIST          G               $NOLI    don't list subsequent source lines

-------------------------------------------------------------------------------

$MACRO(n)        P   $MACRO(50)  $MR      reserve n % of free memory for macros

$NOMACRO         P               $NOMR    reserve all for the symbol table

-------------------------------------------------------------------------------

$MOD51           P   $MOD51      $MO      enable predefined SFR symbols

$NOMOD51         P               $NOMO    disable predefined SFR symbols

-------------------------------------------------------------------------------

$NOBUILTIN       P   list SFR    ---      don't list predefined symbols

-------------------------------------------------------------------------------

$NOTABS          P   use tabs    ---      don't use tabs in list file

-------------------------------------------------------------------------------

$PAGING          P   $PAGING     $PI      enable listing page formatting

$NOPAGING        P               $NOPI    disable listing page formatting

-------------------------------------------------------------------------------

$PAGELENGTH(n)   P   n=64        $PL      set lines per page for listing

-------------------------------------------------------------------------------

$PAGEWIDTH(n)    P   n=132       $PW      set columns per line for listing

-------------------------------------------------------------------------------

$PHILIPS         P   MCS-51      ---      switch on 83C75x family support

-------------------------------------------------------------------------------

$SAVE            G               $SA      save current $LIST/$GEN/$COND state

$RESTORE         G               $RS      restore old $LIST/$GEN/$COND state

-------------------------------------------------------------------------------

$SYMBOLS         P   $SYMBOLS    $SB      create symbol table

$NOSYMBOLS       P               $NOSB    don't create symbol table

-------------------------------------------------------------------------------

$TITLE(string)   G   copyright   $TT      inserts title string into page header

-------------------------------------------------------------------------------

$XREF            P   $NOXREF     $XR      create cross reference

$NOXREF          P               $NOXR    don't create cross reference

The subsequent paragraphs contain detailed explanations of the implemented

controls.

III.8.1 Primary Controls

------------------------

$DATE (string)    Inserts a date string into the list file page header.

If $DATE() is specified, the actual date is inserted.

Date strings will be truncated to a maximum length of

11 characters.

Default is: no date string.

The control has no effect, when the $NOPAGING control has

been specified.

$DEBUG            Includes debug information into the OMF-51 module. When

generating Intel-HEX file output, $DEBUG has no effect.

$NODEBUG          Don't include debug information. (Default!)

$MACRO (n)        Save macro definitions and expand macro calls. (Default!)

Optionally reserve n % of free memory for macro definitions.

(0 <= n <= 100)

Default is n=50.

The control has been implemented for compatibility purposes

only. In ASEM-51 it has no effect except that it cancels

the $NOMACRO control.

$NOMACRO          Don't save macro definitions and don't expand macro calls.

Reserve all free memory for the symbol table.

The control has been implemented for compatibility purposes

only. In ASEM-51, it only suppresses the macro expansion.

$MOD51            Switches on the built-in 8051 special function register

and interrupt symbol definitions. (Default!)

$NOMOD51          Switches off the built-in 8051 special function register

and interrupt symbol definitions. The predefined symbols

??ASEM_51 and ??VERSION cannot be switched off!

$PAGING           Switches on the page formatting in the list file.

(Default!)

$NOPAGING         Switches off the page formatting in the list file.

$PAGELENGTH (n)   Sets the list file page length to n lines.

(12 <= n <= 65535)

Default is n=64.

The control has no effect, when the $NOPAGING control has

been specified.

$PAGEWIDTH (n)    Sets the list file page width to n columns.

(72 <= n <= 255)

Default is n=132.

$PHILIPS          Switches on the Philips 83C75x family support option.

This disables the LJMP, LCALL, and MOVX instructions as

well as the XDATA and XSEG pseudo instructions. Generic

jumps and calls will always assemble to absolute addressing.

$SYMBOLS          Generates the symbol table at the end of the list file.

(Default!)

When the $XREF control is active, $SYMBOLS has no effect!

$NOSYMBOLS        Suppresses the symbol table at the end of the list file.

When the $XREF control is active, $NOSYMBOLS has no effect!

$NOBUILTIN        Suppresses the predefined (built-in) symbols in the symbol

table or cross-reference listing for a better survey.

Only the user-defined symbols are listed.

$NOTABS           Expands all tab characters in the list file output to

blanks.

$XREF             Generates a cross-reference listing instead of a symbol

table. Note that this slightly slows down assembly, and

consumes about 67 % more memory space!

$NOXREF           Generates a symbol table instead of a cross-reference

listing. (Default!)

Examples:   $NOMOD51              ;switch off 8051 SFR symbol definitions

$PAGELENGTH(60)       ;set page length to 60 lines per page

$PW(80)               ;set page width to 80 characters per line

$NOSYMBOLS            ;no symbol table required

$NOTABS               ;printer doesn't support tab characters

$DATE(2. 8. 95)       ;date of latest version

$XREF                 ;generate a cross-reference listing

$ DEBUG NOPAGING      ;include debugging information into OMF-51

;modules, and suppress page formatting

III.8.2 General Controls

------------------------

$COND             List full IFxx .. ELSEIFxx .. ELSE .. ENDIF constructions.

(Default!) The Control is overridden by $NOLIST.

$NOCOND           Don't list lines in false IFxx .. ELSEIFxx .. ELSE .. ENDIF

branches. The Control is overridden by $NOLIST.

$CONDONLY         List lines in true IFxx .. ELSEIFxx .. ELSE .. ENDIF

branches only, without the IFxx, ELSEIFxx, ELSE and ENDIF

statements itself. The Control is overridden by $NOLIST.

$EJECT            Starts a new page in the list file.

The control has no effect, when the $NOPAGING control has

been specified.

$ERROR (string)   Forces an assembly error with a user-defined error message.

This is intended to support configuration management and

can be applied sensefully with conditional assembly only.

$WARNING (string) Outputs a user-defined warning message to the console, and

increments the warning count. This is also intended to ease

configuration management.

$GEN              List macro calls and expanded macros. (Default!)

The listing fully shows the nesting of macro calls.

The Control is overridden by $NOLIST.

$NOGEN            List macro calls only. The expanded macros are not listed.

The Control is overridden by $NOLIST.

$GENONLY          List the expanded macro bodies only. Macro calls and EXITM

statements are not listed.

The Control is overridden by $NOLIST.

$INCLUDE (file)   Includes an external source file into the assembler program

just behind the $INCLUDE statement. If the include file has

not been specified with an absolute path, and it cannot be

found in the default directory, the path specified with the

/INCLUDES (Linux: -i) command line option (if present) is

searched from left to right, and if it cannot be found there

either, the path specified with the environment variable

ASEM51INC (if defined) is searched from left to right as well.

Include files may be nested to any depth.

$LIST             List source code lines. (Default!)

$NOLIST           Do not list source code lines, provided they do not contain

errors, until the next $LIST statement occurs.

$SAVE             Saves the current $LIST/$GEN/$COND state on a $SAVE-stack.

$SAVE statements can be nested to any depth.

$RESTORE          Restores a previously saved $LIST/$GEN/$COND state.

$TITLE (string)   Inserts a title string into the list file page header.

Titles may be truncated according to the specified (or

default) page width.

Default: ASEM-51 copyright information.

The control has no effect, when the $NOPAGING control has

been specified.

Examples:   $NOLIST                ;switch off listing

$INCLUDE (8052.MCU)    ;include 8052 SFR symbol definition file

$LIST                  ;switch on listing

$TITLE (Computer-Controlled Combustion Unit for Motorcycles)

$EJ                    ;new page with new title

$ERROR(invalid configuration: buffer size > external RAM size)

$WARNING(int. RAM doesn't meet minimum stack size requirements)

$SAVE GENONLY CONDONLY ;save old $LIST/$GEN/$COND status, and list

;only source lines that are really assembled

$RESTORE               ;restore previous listing mode

III.9 Predefined Symbols

------------------------

For easy access to the 8051 special function register and interrupt

addresses, ASEM-51 has a number of predefined (built-in) DATA, BIT and

CODE symbols.

These predefined symbols can be switched off with the $NOMOD51 control.

For detailed information on symbols and addresses refer to Appendix C.

For identification of the assembler and its version number, the following

NUMBER type symbols are predefined:

??ASEM_51  =  8051H         ASEM-51

??VERSION  =  0130H         version 1.3

These two symbols can not be switched off!

III.10 Conditional Assembly

---------------------------

Conditional assembly allows to assemble or ignore selected parts of code.

This can be used to keep the code for various program variants in a single

source, to ease configuration control and maintenance. Conditional assembly

is also useful to write fancy macros.

The following fourteen meta instructions have been implemented:

IF     <expr>            ELSEIF     <expr>

IFN    <expr>            ELSEIFN    <expr>

IFDEF  <symbol>          ELSEIFDEF  <symbol>

IFNDEF <symbol>          ELSEIFNDEF <symbol>

IFB    <literal>         ELSEIFB    <literal>

IFNB   <literal>         ELSEIFNB   <literal>

ENDIF                    ELSE

Meta instructions overlay the Intel MCS-51 assembly language, but are not

part of it! C programmers may compare them to C preprocessor commands.

In the subsequent text, IFxx is used as a collective name for the

IF/IFN/IFDEF/IFNDEF/IFB/IFNB instructions. In analogy ELSEIFxx is used as a

collective name for the ELSEIF/ELSEIFN/ELSEIFDEF/ELSEIFNDEF/ELSEIFB/ELSEIFNB

instructions (not including ELSE).

III.10.1 General IFxx Construction

----------------------------------

Simple IFxx ... ENDIF constructions can be used to assemble a number of

enclosed statements only, if a particluar condition is met:

IFxx <condition>

<statement 1>

<statement 2>

.               ;assembled if <condition> is TRUE

.

<statement n>

ENDIF

The statements 1 through n are assembled if <condition> is TRUE, otherwise

they are ignored.

If it should be possible to select two variants of code depending on a

particular condition, this can be done with an IFxx .. ELSE .. ENDIF

construction. If the <condition> in the IFxx statement is TRUE, then

statements 1 to n are assembled and the statements n+1 to n+m are ignored.

IFxx <condition>

<statement 1>

.               ;assembled if <condition> is TRUE

<statement n>

ELSE

<statement n+1>

.               ;assembled if <condition> is FALSE

<statement n+m>

ENDIF

Should <condition> be FALSE, it is exactly vice versa! That means the

statements 1 to n are ignored and the statements n+1 to n+m are assembled.

This works also, if the IFxx or ELSE branches contain no statements at all.

Whenever more than two cases have to be distinguished, a corresponding number

of ELSEIFxx branches can be inserted between the IFxx and the ELSE branch.

In such an IFxx .. ELSEIFxx .. ELSE .. ENDIF construction, only the statements

in the branch with the first TRUE condition are assembled. The statements in

all other branches are ignored.

If none of the conditions is TRUE, only the statements in the ELSE branch

(if any) are assembled.

IFxx <condition 1>

.                ;assembled if <condition 1> is TRUE

.

ELSEIFxx <condition 2>

.                ;assembled if <condition 1> is FALSE,

.                ;and <condition 2> is TRUE

ELSEIFxx <condition 3>

.                ;assembled if <condition 1> and

.                ;<condition 2> are FALSE, and

.                ;<condition 3> is TRUE

.

.

ELSEIFxx <condition n>

.                ;assembled if <condition 1> thru

.                ;<condition n-1> are FALSE and

.                ;<condition n> is TRUE

ELSE

.                ;assembled if <condition 1> thru

.                ;<condition n> are FALSE

ENDIF

IFxx ... ELSEIFxx ... ELSE ... ENDIF constructions may be nested to any depth!

The listing mode of those constructions can be set with the $COND, $NOCOND and

$CONDONLY controls.

III.10.2 IFxx and ELSEIFxx Instructions

---------------------------------------

The particular IFxx instructions are working as follows:

IF <expr>       The IF condition is TRUE, if the expression <expr> is not

equal to 0. The value of <expr> must be known on pass 1!

IFN <expr>      The IFN condition is TRUE, if the expression <expr> is

equal to 0. The value of <expr> must be known on pass 1!

IFDEF <symbol>  The IFDEF condition is TRUE, if the <symbol> is defined in

the program. Forward references to <symbol> are not allowed!

IFNDEF <symbol> The IFNDEF condition is TRUE, if the <symbol> is not defined

in the program. Forward references to <symbol> are not allowed!

IFB <literal>   The IFB (if blank) condition is TRUE, if the <literal> is

empty. <literal> is a string, enclosed in angle brackets.

IFNB <literal>  The IFNB (if not blank) condition is TRUE, if the <literal> is

not empty. <literal> is a string, enclosed in angle brackets.

Although the IFB and IFNB statements are valid also outside of

macros, they can be applied sensefully in macro bodies only.

Usually they are used to decide, whether macro arguments have

been left blank, or not.

The corresponding ELSEIFxx instructions are working respectively.

Example 1:  IF .. ELSE .. ENDIF construction

----------

TARGET EQU 0    ;configuration:  1 for application board

;--------------  0 for evaluation board

IF TARGET

ORG 0         ;program start address of application board

ELSE

ORG 8000H     ;program start address of evaluation board

ENDIF

Currently the program is configured for the evaluation board

version.

Example 2:  IFNDEF .. ELSE .. ENDIF construction

----------

;EVA_537 EQU 0      ;symbol undefined: 80C537 application board

;symbol defined:   80C537 evaluation board

IFNDEF EVA_537

CLOCK EQU 16     ;clock frequency of application board

CSEG  AT  0      ;program start address of application board

ELSE

CLOCK EQU 12     ;clock frequency of evaluation board

CSEG  AT  8000H  ;program start address of evaluation board

ENDIF

Currently the program is configured for the application board

version.

Example 3:  IFB .. ELSE .. ENDIF construction

----------

DECIDE MACRO X, Y

IFB <X&Y>

NOP

NOP

ELSE

DB '&X,&Y'

ENDIF

ENDM

If the above macro is invoked as follows,

DECIDE Nonsense

the parameter X will be replaced by "Nonsense" and the

parameter Y by a zero length string. Thus the IFB literal

becomes <Nonsense>, and the macro will be expanded to:

DB 'Nonsense,'

If the macro will be invoked without arguments,

DECIDE

the parameters X and Y will be replaced by zero length

strings both, and the IFB literal becomes <>. Thus the

macro will be expanded to:

NOP

NOP

Macros are explained in detail in chapter

"III.11 Macro Processing".

Example 4:  IFNDEF .. ELSEIF .. ELSEIF .. ELSE .. ENDIF construction

----------

The symbol BAUDRATE serves to define the UART baudrate:

IFNDEF BAUDRATE

LJMP AUTOBAUD            ;automatic baudrate detection

ELSEIF BAUDRATE EQ 9600

MOV TH1, #0FDH           ;9600 baud

ELSEIF BAUDRATE EQ 1200

MOV TH1, #0E8H           ;1200 baud

ELSE

$ERROR(baudrate not implemented)

ENDIF

If the symbol BAUDRATE is not defined at all, a jump to

the label AUTOBAUD is performed.

If the symbol BAUDRATE is defined with one of the legal

values 9600 or 1200, timer 1 is initialized accordingly.

If the symbol BAUDRATE is defined with another value, a

corresponding user-defined error message is generated.

III.11 Macro Processing

-----------------------

Macros allow to combine basic assembler instructions to "super commands".

For this, macros are defined as blocks of code, which can be used in a

program, wherever it is desired.

However, an advanced macro design with parameters, local symbols and macro

operators, combined with conditional assembly, goes far beyond this basic

functionality!

With only five keywords (MACRO, REPT, ENDM, EXITM, LOCAL) and some control

characters the ASEM-51 macro processor provides a variety of powerful tools.

These five meta instructions are not part of the Intel MCS-51 assembly

language, but overlay it, as already known from conditional assembly.

There are two sorts of macros: callable macros and repeat blocks.

III.11.1 Simple Callable Macros

-------------------------------

Macros must first be defined, before they can be called in a program.

A simple macro definition consists of the macro name, which can be defined

with the keyword MACRO, the macro body, and a final ENDM (end macro)

instruction.

<macro name> MACRO

<body line 1>

<body line 2>

.

.

<body line m>

ENDM

The macro name must be a valid, unique symbol. It cannot be redefined later.

Keywords cannot be used as macro names.

The macro body may comprise any number of lines. Body lines may be all kinds

of assembler instructions, pseudo instructions, controls, meta instructions,

macro calls and even further macro definitions.

The macro body and the whole macro definition is terminated with the ENDM

instruction.

Macros must be defined, before they can be called. Forward references to

macros are not allowed. Once defined, a macro can be called by its name in

the subsequent program as often as desired. Whenever a macro is called, the

macro body will be "inserted" into the program and then assembled as normal

source lines. This process is called macro expansion.

Example:        MY_FIRST MACRO  ;definition

--------        MOV A,#42

ADD A,R5

ENDM

MY_FIRST        ;call

After the call of the macro MY_FIRST, the body lines

MOV A,#42

ADD A,R5

are inserted into the program and assembled.

III.11.2 Macro Parameters

-------------------------

Callable macros may have parameters, to allow more flexible use.

The names of the formal parameters are specified in the macro definition

behind the keyword MACRO, separated by commas. All parameter names of a

macro must be different, valid symbols. Keywords cannot be used as

parameter names. Macros may have any number of parameters, as long as they

fit on one line. Parameter names are local symbols, which are known within

the macro only. Outside the macro they have no meaning!

<macro name> MACRO <parameter 1>, <parameter 2>, ... ,<parameter n>

<body line 1>

<body line 2>

.

.

<body line m>

ENDM

When called, actual arguments can be passed to the macro. The arguments

must be separated by commas. Valid macro arguments are

1. arbitrary sequences of printable characters, not containing blanks,

tabs, commas, or semicolons

2. quoted strings (in single or double quotes)

3. single printable characters, preceded by '!' as an escape character

4. character sequences, enclosed in literal brackets < ... >, which may

be arbitrary sequences of valid macro arguments (types 1. - 4.),

blanks, commas and semicolons

5. arbitrary sequences of valid macro arguments (types 1. - 4.)

6. expressions preceded by a '%' character

Note: The keywords MACRO, EQU, SET, CODE, DATA, IDATA, XDATA, BIT, and

the ':' character cannot be passed as the first macro argument,

because they always start a symbol definition!

Therefore they must be enclosed in literal brackets < ... >.

During macro expansion, these actual arguments replace the symbols of the

corresponding formal parameters, wherever they are recognized in the macro

body. The first argument replaces the symbol of the first parameter, the

second argument replaces the symbol of the second parameter, and so forth.

This is called substitution.

Without special assistance, the assembler will not recognize a parameter

symbol if it

- is part of another symbol

- is contained in a quoted string

- appears in commentary

Example 1:      MY_SECOND MACRO CONSTANT, REGISTER

----------      MOV A,#CONSTANT

ADD A,REGISTER

ENDM

MY_SECOND 42, R5

After calling the macro MY_SECOND, the body lines

MOV A,#42

ADD A,R5

are inserted into the program, and assembled.

The parameter names CONSTANT and REGISTER have been

replaced by the macro arguments "42" and "R5".

The number of arguments, passed to a macro, can be less (but not greater)

than the number of its formal parameters. If an argument is omitted, the

corresponding formal parameter is replaced by an empty string.

If other arguments than the last ones are to be omitted, they can be

represented by commas.

Example 2:      The macro OPTIONAL has eight formal parameters:

----------

OPTIONAL MACRO P1,P2,P3,P4,P5,P6,P7,P8

.

.

<macro body>

.

.

ENDM

If it is called as follows,

OPTIONAL 1,2,,,5,6

the formal parameters P1, P2, P5 and P6 are replaced by the

arguments 1, 2, 5 and 6 during substitution. The parameters

P3, P4, P7 and P8 are replaced by a zero length string.

For more flexible macro design, there must be a possibility to recognize

empty macro arguments, and to branch the macro expansion accordingly.

This can be performed with conditional assembly, using the IFB and IFNB

meta instructions. (See chapter "III.10.2 IFxx and ELSEIFxx Instructions".)

III.11.3 Repeat Macros

----------------------

Repeat macros don't have a macro name, and therefore cannot be called

multiple times. They are always expanded immediately after their definition.

During expansion, their macro body is repeated n times (0 <= n <= 65535).

Repeat macros start with the keyword REPT, followed by an expression, which

must be known on pass 1. In analogy to callable macros, there is a macro

body, which must be terminated with an ENDM instruction:

REPT <expression>

<body line 1>

<body line 2>

.

.

<body line m>

ENDM

The expression value specifies how many times the macro body is to be

repeated. Since repeat macros start with the keyword REPT, they are

sometimes also called "REPT blocks".

Example:        REPT 5

--------        NOP

ENDM

This REPT block will expand to five NOP instructions

immediately after its definition:

NOP

NOP

NOP

NOP

NOP

III.11.4 Local Symbols

----------------------

Local symbols are symbols, which are only known within a macro body, but not

outside the macro. Symbols that are defined for the whole program, will

subsequently be called "global symbols" for better understanding.

We are already familiar with a special case of local symbols: formal macro

parameters. They appear in the macro definition only. Since they are

substituted during macro expansion, we don't have further problems with them.

But what happens with symbols that are defined in a macro body?

Example 1:      The following simple macro is intended to read a character

----------      from the 8051 UART, and to return it in A:

RECEIVE MACRO

UARTIN: JNB RI,UARTIN

MOV A,SBUF

CLR RI

ENDM

This will work only once! If the macro RECEIVE is called

multiple times, the label UARTIN will be multiply defined.

This can be solved by simply declaring the symbol UARTIN local.

For this, the LOCAL statement has been introduced. After the keyword

LOCAL, a list of local symbols can be specified, separated by commas.

These symbols will only be valid inside the macro that contains the LOCAL

statement. LOCAL statements may only be placed directly after the MACRO

or REPT statement, preceding the first body line. They may contain any

number of local symbols. The macro body may be preceded by an arbitrary

number of LOCAL statements.

Local symbols must be valid symbols, unique within the macro, and different

from the formal parameters (if any). Keywords cannot be used as local symbol

names. If a local symbol has the same name as a global symbol, the local

scope takes precedance during substitution.

When a macro is expanded, its local symbols are always substituted: the

formal parameters are replaced by the macro arguments, and the local symbols

that have been declared in a LOCAL statement are replaced by unique, global

symbol names, which the assembler generates during every expansion. These

have always the format ??xxxx, where xxxx is a unique symbol number.

Example 2:      After a redesign of our previous macro RECEIVE using

----------      local symbols, it is looking as follows:

RECEIVE MACRO

LOCAL UARTIN

UARTIN: JNB RI,UARTIN

MOV A,SBUF

CLR RI

ENDM

Enhanced as shown above, the macro will work correctly,

as often as desired. When RECEIVE is called for the first

time, the local symbol UARTIN will be replaced by ??0000,

??0000: JNB RI,??0000

MOV A,SBUF

CLR RI

when it is called for the second time, UARTIN will be

replaced by ??0001, and so on:

??0001: JNB RI,??0001

MOV A,SBUF

CLR RI

However, it is recommended not to define global symbols in the format ??xxxx,

to avoid name conflicts with substituted local symbols from expanded macros.

III.11.5 Macro Operators

------------------------

There are some special control characters, which are very useful for macro

definition, call and expansion:

;;      Macro commentary:

Normally, comments in body lines are also contained in the expanded

lines. If a commentary begins with ';;' however, it is not stored

during macro definition. Therefore, it doesn't consume memory space,

and appears in the list file in the macro definition only, but not

in the expanded lines.

!       Literal operator:

If the escape character '!' precedes another printable character in

a macro argument, the assembler is forced to treat that character

literally. This means it will be passed to the macro, even if it is

a control character, while the literal operator itself is removed.

< >     Literal brackets:

If a macro argument is intended to contain separation or control

characters, it must be enclosed in literal brackets < ... > to pass

it to the macro as one argument string, while the outermost pair of

brackets is removed. Literal brackets can be nested to any depth.

%       Evaluation:

If a macro argument is preceded by the evaluation operator '%', it

is interpreted as an expression, which will be evaluated before it

is passed to the macro. The actual argument string will not be the

expression itself, but a decimal ASCII representation of its value.

The expression must be known on pass 1.

&       Substitution:

The '&' character separates parameter names (local symbols) from

surrounding text. Outside quoted strings and commentary it serves

only as a general separation character. This applies always when

a local symbol directly precedes or follows another alphanumeric

string. Inside quoted strings and commentary, a local symbol must

be preceded by '&' if it is to be substituted there.

During every macro expansion, the assembler removes exactly one '&'

from every sequence of '&' characters. This allows for example, to

define a nested macro inside a macro body, which also uses the

substitution operator '&': one writes simply '&&'!

Example 1:      The commentary should only be visible in the definition

----------      of the macro LICENSE:

LICENSE MACRO

DB 'Copyright'  ;;legal stuff

ENDM

When called, the expanded macro body is looking

like this in the list file:

DB 'Copyright'

Example 2:      SPECIAL !;

----------

passes a semicolon to the macro SPECIAL as a literal

argument. This could also be done with

SPECIAL <;>

Example 3:      The macro CONST defines a 16-bit constant in ROM:

----------

CONST MACRO NUMB

DW NUMB

ENDM

If it is called as shown below,

CONST 0815H+4711-42

the parameter NUMB would be substituted as follows:

DW 0815H+4711-42

If the same macro argument is preceded by a '%' however,

CONST %0815H+4711-42

the substitution will result in:

DW 6738

Example 4:      During substitution, both arguments of the macro CONCAT

----------      should form a seamless symbol name:

CONCAT MACRO NAM, NUM

MOV R3,#0

NAM&NUM: DJNZ R3,NAM&NUM

ENDM

When CONCAT is called as follows,

CONCAT LABEL, 08

the parameters NAM and NUM are substituted during macro

expansion as shown below:

MOV R3,#0

LABEL08: DJNZ R3,LABEL08

III.11.6 Premature End of a Macro Expansion

-------------------------------------------

Sometimes it is useful, if a macro expansion can be terminated, before

the end of the macro body is reached. This can be forced with the EXITM

(exit macro) instruction. However, this makes sense in conjunction with

conditional assembly only.

Example:        FLEXIBLE MACRO QUANTITY

--------        DB 'Text'

IF QUANTITY LE 255

EXITM

ENDIF

DW QUANTITY

ENDM

The macro FLEXIBLE always has to insert the string 'Text'

into the CODE space. After that, it should insert a 16-bit

constant only, if the numerical value of the parameter

QUANTITY is greater than 255.

Otherwise the macro expansion should be terminated with

EXITM before. If the macro is called as follows,

FLEXIBLE 42

it will be expanded to

DB 'Text'

in list mode $GENONLY/$CONDONLY.

However, if it is called like this,

FLEXIBLE 4711

it will be expanded to:

DB 'Text'

DW 4711

When a macro expansion is terminated with EXITM, all IFxx constructions that

have been opened within the macro body so far, are closed.

Of course macro bodies may also contain control statements. If an include

file is inserted into a macro body with a $INCLUDE control, and this include

file, or a nested include file, contains an EXITM instruction, all include

file levels up to the next macro level are closed at this point, and the

expansion of that macro is terminated immediately.

III.11.7 Nested and Recursive Macro Calls

-----------------------------------------

Macro bodies may also contain macro calls, and so may the bodies of those

called macros, and so forth.

If a macro call is seen throughout the expansion of a macro, the assembler

starts immediately with the expansion of the called macro. For this, its

its expanded body lines are simply inserted into the expanded macro body of

the calling macro, until the called macro is completely expanded. Then the

expansion of the calling macro is continued with the body line following

the nested macro call.

Example 1:      INSIDE MACRO

----------      SUBB A,R3

ENDM

OUTSIDE MACRO

MOV A,#42

INSIDE

MOV R7,A

ENDM

In the body of the macro OUTSIDE, the macro ISIDE is called.

If OUTSIDE is called (and the list mode is set to $GENONLY),

one gets something like the following expansion:

Line  I  Addr  Code            Source

15+ 1  0000  74 2A           MOV A,#42

17+ 2  0002  9B              SUBB A,R3

18+ 1  0003  FF              MOV R7,A

Since macro calls can be nested to any depth (while there is free memory),

the macro expansion level is shown in the I-column of the list file.

Since macro and include file levels can be nested in arbitrary sequence and

depth, the nesting level is counted through all macro and include file

levels regardless. For better distinction, the character following the

global line number is ':' for include file levels, and '+' for macro levels.

If macros are calling themselves, one speaks of recursive macro calls.

In this case, there must be some stop criterion, to prevent the macro of

calling itself over and over until the assembler is running out of memory!

Here again, conditional assembly is the solution:

Example 2:      The macro COUNTDOWN is to define 16-bit constants from

----------      1 thru n in descending order in ROM. n can be passed to

the macro as a parameter:

COUNTDOWN MACRO DEPTH

IF DEPTH GT 0

DW DEPTH

COUNTDOWN %DEPTH-1

ENDIF

ENDM

If COUNTDOWN is called like this,

COUNTDOWN 7

something like the following macro expansion results

(in list mode $GENONLY/$CONDONLY):

Line  I  Addr  Code            Source

16+ 1  0000  00 07           DW 7

19+ 2  0002  00 06           DW 6

22+ 3  0004  00 05           DW 5

25+ 4  0006  00 04           DW 4

28+ 5  0008  00 03           DW 3

31+ 6  000A  00 02           DW 2

34+ 7  000C  00 01           DW 1

After the Dark Ages, when the dust was settling and the sun broke through

the gloom, computer science discovered the method of recursive programming.

There was no doubt that this was the SOLUTION!

And the computer scientists started to explain this to the students.

But it seemed that the students didn't get it. They always complained

that recursive calculation of n! is a silly example indeed.

All the scientists felt stronly that there was still something missing.

After 10 more years of hard research work, they also found the PROBLEM:

Example 3:      The Towers of Hanoi

----------

There are three vertical sticks on the table. On stick 1

there are n discs with different diameters and a hole in

the middle, the smallest disc on top, the biggest on the

bottom.

Stick 1          Stick 2          Stick 3

|                |                |

Disc 1       ===|===             |                |

Disc 2     =====|=====           |                |

Disc n   =======|=======         |                |

__________|________________|________________|__________

///////////////////////////////////////////////////////

The PROBLEM is to transfer the tower of discs from stick 1

to stick 2 with a minimum of moves. But only the topmost

disc on a tower may be moved at one time, and no disc may

be layed on a smaller disc. Stick 3 may be used for scratch

purposes. This is a SOLUTION with ASEM-51 macros:

;The Towers of Hanoi

$GENONLY CONDONLY

DISCS EQU 3     ;number of discs

HANOI MACRO n, SOURCE, DESTINATION, SCRATCH

IF n > 0

HANOI %(n-1), SOURCE, SCRATCH, DESTINATION

;   move topmost disc from stick &SOURCE to stick &DESTINATION

HANOI %(n-1), SCRATCH, DESTINATION, SOURCE

ENDIF

ENDM

HANOI DISCS, 1, 2, 3

END

The recursive macro HANOI generates an instruction manual

for the PROBLEM, where the instructions appear as comment

lines in the list file. The symbol DISCS must be set to

the desired number of discs. If HANOI is called like this,

HANOI 3, 1, 2, 3

the following "instruction manual" is generated:

27+ 3       ;   move topmost disc from stick 1 to stick 2

35+ 2       ;   move topmost disc from stick 1 to stick 3

44+ 3       ;   move topmost disc from stick 2 to stick 3

53+ 1       ;   move topmost disc from stick 1 to stick 2

64+ 3       ;   move topmost disc from stick 3 to stick 1

72+ 2       ;   move topmost disc from stick 3 to stick 2

81+ 3       ;   move topmost disc from stick 1 to stick 2

The GENONLY and CONDONLY controls ensure that the table

doesn't contain all the macro calls and IF constructions.

Exercise 1:     Modify the macro HANOI so that it is generating a move

table in ROM, which could directly be used as an input

for an 8051-controlled robot-arm that really plays the

game with 3 real sticks and n real discs.

n

Exercise 2:     Prove that the minimum number of moves is 2 - 1.   ;-)

III.11.8 Nested Macro Definitions

---------------------------------

A macro body may also contain further macro definitions. However, these

nested macro definitions aren't valid until the enclosing macro has been

expanded! That means, the enclosing macro must have been called, before

the nested macros can be called.

Example 1:      A macro, which can be used to define macros with arbitrary

----------      names, may look as follows:

DEFINE MACRO MACNAME

MACNAME MACRO

DB 'I am the macro &MACNAME.'

ENDM

ENDM

In order not to overload the example with "knowhow",   :-)

the nested macro only introduces itself kindly with

a suitable character string in ROM. The call

DEFINE Obiwan

would define the macro

Obiwan MACRO

DB 'I am the macro Obiwan.'

ENDM

and the call

DEFINE Skywalker

would define the following macro:

Skywalker MACRO

DB 'I am the macro Skywalker.'

ENDM

Example 2:      A macro is to insert a variable number of NOPs into the

----------      program. For this, a macro with a nested REPT block seems

to be best-suited:

REPEAT MACRO NOPS

REPT NOPS

NOP

ENDM

ENDM

The macro call

REPEAT 4

results in something like that:

Line  I  Addr  Code            Source

9+ 1        N      0004     REPT 4

10+ 1                        NOP

11+ 1                        ENDM

12+ 2  0000  00              NOP

13+ 2  0001  00              NOP

14+ 2  0002  00              NOP

15+ 2  0003  00              NOP

III.11.9 Representation in the List File

----------------------------------------

Sometimes macro expansions tend to produce much more listing lines than

resulting code. To list or not to list - that is the question!

The requirements to either get a better overall view or more detailed

information may vary in different development phases or program sections.

To always get the best results, a number of general controls has been

introduced, which influence the representation of macro expansions and IFxx

constructions in the list file (see chapter "III.8 Assembler Controls"):

Control   Type   Default   Abbreviation            Meaning

------------------------------------------------------------------------------

$GEN        G    $GEN      $GE           list macro calls and expansion lines

$NOGEN      G              $NOGE         list macro calls only

$GENONLY    G              $GO           list expansion lines only

------------------------------------------------------------------------------

$COND       G    $COND     ---           list full IFxx .. ENDIF constructions

$NOCOND     G              ---           don't list lines in false branches

$CONDONLY   G              ---           list assembled lines only

------------------------------------------------------------------------------

$SAVE       G              $SA           save current $LIST/$GEN/$COND state

$RESTORE    G              $RS           restore old $LIST/$GEN/$COND state

IV. Compatibility with the Intel Assembler

==========================================

With their cross assembler ASM51, Intel has defined and implemented a

suitable assembly language for the MCS-51 family, which has always been

the only real standard in the 8051 world.

Unfortunately, Intel has announced the "end of life" of ASM51 (final

version 2.3) and all the other Intel MCS-51 development tools to the

end of 1993.

The ASEM-51 assembly language is a subset of the Intel standard that

guarantees maximum compatibility with existing 8051 assembler sources.

It implements all 8051 instruction mnemonics as well as a rich and useful

subset of the Intel pseudo instructions and assembler controls.

IV.1 Restrictions

-----------------

Since ASEM-51 generates an Intel-HEX file (or absolute OMF-51) output

instead of relocatable object modules, the whole source code of an 8051

application program has to reside in one single file. Consequently all

pseudo instructions that deal with relocatable segments or external or

public symbols, have not been implemented:

PUBLIC

EXTRN

SEGMENT

RSEG

Intel-style macros are not supported! (Thus the '%' character can be used in

comments.)

Up to now only the following assembler controls and their abbreviations have

been implemented:

|  primary controls         abbrev. |  general controls    abbrev.

---------+-----------------------------------+-----------------------------

|  $DATE (<string>)          $DA    |  $EJECT               $EJ

|  $DEBUG                    $DB    |  $GEN                 $GE

|  $NODEBUG                  $NODB  |  $NOGEN               $NOGE

|  $MACRO (<percent>)        $MR    |  $GENONLY             $GO

|  $NOMACRO                  $NOMR  |  $INCLUDE (<file>)    $IC

Intel-   |  $MOD51                    $MO    |  $LIST                $LI

|  $NOMOD51                  $NOMO  |  $NOLIST              $NOLI

controls |  $PAGING                   $PI    |  $SAVE                $SA

|  $NOPAGING                 $NOPI  |  $RESTORE             $RS

|  $SYMBOLS                  $SB    |  $TITLE (<string>)    $TT

|  $NOSYMBOLS                $NOSB  |

|  $PAGELENGTH (<lines>)     $PL    |

|  $PAGEWIDTH (<columns>)    $PW    |

---------+-----------------------------------+-----------------------------

|  $NOBUILTIN                -----  |  $COND                -----

ASEM-51  |  $NOTABS                   -----  |  $NOCOND              -----

controls |  $PHILIPS                  -----  |  $CONDONLY            -----

|                                   |  $ERROR (<string>)    -----

|                                   |  $WARNING (<string>)  -----

IV.2 Extensions

---------------

Assembler controls need not start in column 1, but may be preceded by any

number of blanks and tabs. Primary controls may also be preceded by comment

lines and $INCLUDE statements, provided the corresponding include files are

only containing other control statements and commentary.

The source file may contain blank and comment lines behind the END statement.

Character strings may also be enclosed in double quotes.

The DATA symbol for the special function register PCON is predefined.

The bit operator '.' is legal in all expressions, not only in those that

have to match the segment type BIT.

ASEM-51 introduces a set of meta instructions, which overlay the Intel MCS-51

assembly language, but are not part of it!

The meta instructions IFxx, ELSEIFxx, ELSE, and ENDIF allow conditional

assembly, while the meta instructions MACRO, REPT, ENDM, EXITM, and LOCAL

(and some control characters) form a powerful macro processing language.

For detailed information on meta instructions see chapters

"III.10 Conditional Assembly" and "III.11 Macro Processing".

IV.3 Further Differences

------------------------

To make semantics unique, especially the precedence of unary operators in

expressions is slightly different. Furthermore, expressions with a bit

operation "." evaluate to a BIT type result, not to NUMBER. The segment

type of symbols that are defined with EQU or SET evaluates always to NUMBER.

Otherwise it might be difficult in some cases, to force the definition of

typeless symbols. This is described in detail in chapters "III.4 Expressions"

and "III.7 Segment Type".

Except in DB instructions, the zero length string constant '' is illegal.

The $NOMOD51 control disables also the predefined CODE addresses.

The special assembler symbols AR0...AR7 are predefined for bank 0 before

the first USING statement occurs.

V. List File Format

===================

The ASEM-51 list file format has been designed to give the user as much

information about the generated code as possible.

Besides the source code listed, there are five basic layout structures

in the listing:

- the page header

- the file header

- the line headings

- the error diagnosis

- the symbol table or cross-reference listing

Normally, every page of the listing starts with a page header as shown below:

ASEM-51 V1.3              Copyright (c) 2002 by W.W. Heinz               PAGE 1

It identifies the assembler, contains the copyright information and shows the

actual page number at the right margin. After the page header, source lines

are output in the list file format. When the maximum number of lines per page

is reached, another page header is output after a form feed character.

If your printer doesn't support form feeds, the page header can be

suppressed with the $NOPAGING control. The number of lines per page can be

adjusted to the paper format with the $PAGELENGTH control. The width of the

page header (and all other lines) can be set with the $PAGEWIDTH control.

The file header appears only on the first page. It identifies the assembler,

lists all input and output files and marks the columns for the line headings.

A typical file header is looking as shown below:

MCS-51 Family Macro Assembler   A S E M - 5 1   V 1.3

=====================================================

Source File:    demo.a51

Object File:    demo.hex

List File:      demo.lst

Line  I  Addr  Code            Source

Directly after the file header starts the listing of the source code lines.

Every source code line is preceded by a line heading. The line heading

consists of four columns: line number, include file or macro level, line

address, and generated code.

By default the line headings contain tab characters to save disk space.

If your printer or file browser doesn't support tabs, they can be expanded

to blanks with the $NOTABS control.

The column "Line" contains the global line number. It is not necessarily

the local line number within the particular source file, but a global line

number that is counted over the main source, all include files, and all

macro expansion lines.

Since include files and macros can be nested arbitrarily, the global line

number is terminated by a ':' character for the main source and all include

file levels, and with a '+' character for macro expansion levels.

The column "I" flags the level of include file or macro nesting. In the main

source, this column is empty. The first include file gets level 1. If this

include file includes another include file, this one gets level 2, and so on.

This is also valid for nested macro calls. If a macro is called in the main

source, its expansion lines get level 1. If this macro calls another one, it

gets level 2, and so forth.

Include file and macro levels can be nested in any sequence and to any depth!

The column "Addr" shows the start address of the listed line in the currently

active segment (8051 address space). All addresses are represented as hex

numbers. The addresses in the CODE and XDATA segments are four-digit numbers.

Addresses in all other segments are two-digit numbers. For lines that cannot

be assigned to a particular segment, the "Addr" field is left blank.

The "Code" column may contain up to four bytes of generated code, which is

sufficient for all 8051 instructions. The code is listed in hex byte

quantities starting from the left margin of the "Code" column.

However, the code generated for DB and DW instructions may be longer than

four bytes. In these cases, the source code line is followed by additional

line headings until the whole code of the line is listed.

The "Code" column does not always contain code that consumes space in the

8051 CODE segment. In contrast to many other assemblers, ASEM-51 lists the

evaluation results of all expressions that may appear in pseudo instructions

or assembler controls. These values are listed in hex representation at the

right margin of the "Code" column. The segment type of those expressions is

flagged with one single character at the left margin of the "Code" column:

C         CODE

D         DATA

I         IDATA

X         XDATA

B         BIT

N         typeless number

R         register

The "Source" column finally contains the original source code line.

A typical source code listing is looking as follows:

Line  I  Addr  Code            Source

1:                                  ;A sample List File Demo Program

2:                                  ;-------------------------------

3:                          $NOMOD51                 ;no 8051 SFR

4:          N      004F     $PAGEWIDTH (79)          ;79 columns per line

5:                          $NOTABS                  ;expand tabs

6:          N        90             P1    DATA 090H  ;port 1 address

7:          B        93             INPUT BIT  P1.3  ;pulse input

8:

9:          N      8000             ORG  08000H      ;set location counter

10:    8000  80 20                   SJMP START       ;jump to start address

11:

12:    8002  01 07                   DB  1,7          ;define bytes

13:    8004  00 02 00 0C             DW  2,12,9       ;define words

8008  00 09

14:    800A  63 6F 66 66             DB  'coffeeright (c) 2002',0    ;string

800E  65 65 72 69

8012  67 68 74 20

8016  28 63 29 20

801A  32 30 30 32

801E  00

15:    801F  N      0003             DS  3            ;define space

16:

17:    8022  75 30 00        START:  MOV COUNT,#0     ;reset counter

18:    8025  30 93 FD        LLEVEL: JNB INPUT,LLEVEL ;wait for high

19:    8028  20 93 FD        HLEVEL: JB  INPUT,HLEVEL ;wait for low

20:    802B  05 30                   INC COUNT        ;count pulse

21:    802D  80 F6                   JMP LLEVEL       ;next pulse

22:

23:          N        30             DSEG AT 030H     ;internal RAM

24:      30  N        01     COUNT:  DS 1             ;counter variable

25:

26:                                  END

If an error is detected in a source line, its position is flagged with a ^

character as good as possible, and a comprehensive error message is inserted.

This is looking as shown below:

17:    8022  75 30 00        START:  MOV COUNT,#0     ;reset counter

18:    8025  30 93 FD        LLEVEL: JNB INPUT,LLEVEL ;wait for high

19:    8028  20 93 00        HLEVEL: JB  INPUT,HLEUEL ;wait for low

^

@@@@@ symbol not defined @@@@@

20:    802B  05 30                   INC COUNT        ;count pulse

21:    802D  80 F6                   JMP LLEVEL       ;next pulse

The error diagnosis at the end of program lists the register banks used,

and the total number of errors detected throughout the assembly:

register banks used:  0, 1, 3

187 errors detected

A register bank counts as "used", if the program had switched to that

bank with a USING instruction, or one of the special assembler symbols

AR0 ... AR7 has been used, while the bank was active. The message

register banks used:  ---

means, that no bank has been used explicitly, and that the program

code may, but need not, be register bank independent.

After the source code listing and error diagnosis, the symbol table or

cross-reference listing starts. By default, a symbol table is generated.

The symbol table lists all the symbols of a program in alphabetical order

with their symbol name, segment type, hex value and first definition line.

Predefined symbols are listed without a definition line number.

The symbol table listing can be suppressed with the $NOSYMBOLS control.

A typical symbol table listing is looking as shown below:

L I S T   O F   S Y M B O L S

=============================

SYMBOL                            TYPE     VALUE        LINE

------------------------------------------------------------

AKKUM                             REGISTER     A          38

COUNT                             DATA        30          47

HLEVEL                            CODE      802E          35

INPUT                             BIT         93          12

LLEVEL                            CODE      802B          34

MY_PROGRAM                        MODULE                  14

P1                                DATA        90

QUANT                             NUMBER    0013          22

RECEIVE                           MACRO                    5

SP                                DATA        81

STACK                             IDATA       80          17

START                             CODE      8022          31

VOLTDC                            XDATA     D785          50

If the $XREF control is specified, a cross-reference listing is generated

instead of a symbol table. The corresponding cross-reference listing for

the symbol table above is looking as follows:

C R O S S - R E F E R E N C E - L I S T I N G

=============================================

SYMBOL                           TYPE     VALUE     DEFINED  REFERENCED

-------------------------------------------------------------------------------

AKKUM                            REGISTER     A          38          42      43

COUNT                            DATA        30          47          32      40

43      44

HLEVEL                           CODE      802E          35          35

INPUT                            BIT         93          12          34      35

LLEVEL                           CODE      802B          34          34      41

MY_PROGRAM                       MODULE                  14

P1                               DATA        90                      12

QUANT                            NUMBER    0007          22          44

NUMBER    0013          37

RECEIVE                          MACRO                    5

SP                               DATA        81                      31

STACK                            IDATA       80          17          31

START                            CODE      8022          31          24

TRASH                            undef.    ----                      42

VOLTDC                           XDATA     D785          50          33

It lists all the symbols of the program in alphabetical order, with their

symbol name, all definitions including definition lines, segment types, and

numerical values. Furthermore, all symbol references are listed as well.

The SYMBOL column contains the symbol name, while the columns TYPE, VALUE,

and DEFINED may contain the segment types, numerical values, and definition

lines of one, more, or no symbol defintions.

Register symbols have the symbol type "REGISTER", module names have the symbol

type "MODULE", macro names have the symbol type "MACRO", and symbols that have

been referenced but not defined, are flagged with "undef." in the TYPE column.

Starting from column REFERENCED up to the right margin, there is a number of

columns (depending on the page width), containing all line numbers of symbol

references (if any).

The cross-reference listing does not distinguish, whether multiple definitions

of, or references to a particular symbol are legal or not. For this, refer to

the error messages in the source listing.

VI. Support of 8051 Derivatives

===============================

Today a large number of 8051 derivatives is available that grows almost

monthly! They all use the same instruction set of the MCS-51 processor core,

but are different in peripheral components, to cover a wide range of

applications. The difference for the assembly language programmer is mainly

the varying set of special function registers and interrupt addresses.

It is always good practice to use the same SFR names in a microcontroller

application program that the manufacturer of the derivative used has defined.

For this the processor definition files *.MCU are provided. They all are

include files with the special function register definitions of a particular

8051 derivative. However, the predefined symbols of ASEM-51 must be switched

off prior to including the SFR definitions of another derivative as shown

below:

$NOMOD51

$INCLUDE (80C515.MCU)

This would switch off the predefined symbols of the 8051 and include the

register definitions of the 80C515 or 80C535 respectively.

Hence it is easy for the user to adapt ASEM-51 to a brandnew 8051 derivative!

All what he has to do is to write a corresponding include file with the SFR

definitions derived from the manufacturer's data sheet.

The name of every processor definition file is corresponding to the ROM

version of a particular derivative. Of course it also applies to the EPROM,

EEPROM, flash, and ROM-less versions (if any) of that derivative.

By the way, the file 8051.MCU provided contains exactly the predefined

symbols of ASEM-51, because its internal symbol table has been generated

from it!

To switch ASEM-51 to the reduced instruction set of the Philips 83C75x

family of microcontrollers, the $PHILIPS control can be used.

Currently the following processor definition files are provided with ASEM-51:

Name        Manufacturer    Versions

---------------------------------------------------------------------

8051.MCU    Intel           8051, 8031, 8751BH

(and others)    8051AH, 8031AH, 8751H, 8051AHP, 8751H-8

80C51BH, 80C31BH, 87C51, 80C51BHP

Atmel           89C51, 89LV51, 87LV51, 80F51, 87F51

8052.MCU    Intel           8052AH, 8032AH, 8752BH

SIEMENS         80513, 8352-5

80C52.MCU    Intel           80C52, 80C32, 87C52,

80C54, 87C54, 80C58, 87C58

83C51FX.MCU    Intel           83C51FA, 80C51FA, 87C51FA

83C51FB, 87C51FB, 83C51FC, 87C51FC

83C51R.MCU    Intel           83C51RA, 80C51RA, 87C51RA,

83C51RB, 87C51RB, 83C51RC, 87C51RC

83C51KB.MCU    Intel           83C51KB

83C51GB.MCU    Intel           83C51GB, 80C51GB, 87C51GB

83C151.MCU    Intel           83C151SB, 87C151SB, 80C151SB

83C151SA, 87C151SA

83C152.MCU    Intel           80C152JA, 83C152JA, 80C152JB

80C152JC, 83C152JC, 80C152JD

83C452.MCU    Intel           83C452, 80C452

8044.MCU    Intel           8044AH, 8344AH, 8744AH

83931HA.MCU    Intel           83931HA, 80931HA

83931AA.MCU    Intel           83931AA, 80931AA

80512.MCU    SIEMENS         80512, 80532

80515.MCU    SIEMENS         80515, 80535, 80515K, 83515-4

80C515.MCU    SIEMENS         80C515, 80C535, 83C515H

83C515A.MCU    SIEMENS         83C515A-5, 80C515A

80C517.MCU    SIEMENS         80C517, 80C537

C501.MCU    Infineon        C501-1R, C501-L

C502.MCU    Infineon        C502-2R, C502-L

C503.MCU    Infineon        C503-1R, C503-L

C504.MCU    Infineon        C504-2R, C504-L

C505.MCU    Infineon        C505-2R, C505-L

C505C.MCU    Infineon        C505C-2R, C505C-L

C505A.MCU    Infineon        C505A-4E, C505A-L

C505CA.MCU    Infineon        C505CA-4E, C505CA-L

C505L.MCU    Infineon        C505L

C508.MCU    Infineon        C508-4R, C508-4E

C509.MCU    Infineon        C509-L

C511.MCU    Infineon        C511, C511A

C513.MCU    Infineon        C513, C513A, C513A-H

C513AO.MCU    Infineon        C513AO

C515.MCU    Infineon        C515-L, C515-1R

C515A.MCU    Infineon        C515A-L, C515A-4R

C515C.MCU    Infineon        C515C-8R

C517A.MCU    Infineon        C517A-L, C517A-4R, 83C517A-5, 80C517A

C540U.MCU    Infineon        C540U

C541U.MCU    Infineon        C541U

C868.MCU    Infineon        C868-1R, C868-1S

83C451.MCU    Philips         83C451, 80C451, 87C451

83C528.MCU    Philips         83C528, 80C528, 87C528, 83C524, 87C524

83CE528, 80CE528, 89CE528

83C550.MCU    Philips         83C550, 80C550, 87C550

83C552.MCU    Philips         83C552, 80C552, 87C552

83C562.MCU    Philips         83C562, 80C562

83C652.MCU    Philips         83C652, 80C652, 87C652

83C654, 87C654, 83CE654, 80CE654

83C750.MCU    Philips         83C750, 87C750

83C751.MCU    Philips         83C751, 87C751

83C752.MCU    Philips         83C752, 87C752

83C754.MCU    Philips         83C754, 87C754

83C851.MCU    Philips         83C851, 80C851

83C852.MCU    Philips         83C852

87LPC762.MCU    Philips         87LPC762

87LPC768.MCU    Philips         87LPC768

80C32X2.MCU    Philips         80C31X2, 80C32X2,

80C51X2, 80C52X2, 80C54X2, 80C58X2,

87C51X2, 87C52X2, 87C54X2, 87C58X2,

89C51X2, 89C52X2, 89C54X2, 89C58X2

80C521.MCU    AMD             80C521, 80C541, 87C521, 87C541, 80C321

80C324.MCU    AMD             80C324

83C154.MCU    OKI             83C154, 80C154, 85C154VS

83C154S.MCU    OKI             83C154S, 80C154S, 85C154HVS

80C310.MCU    DALLAS          80C310

80C320.MCU    DALLAS          80C320, 87C320, 80C323, 87C323

80C390.MCU    DALLAS          80C390

87C520.MCU    DALLAS          87C520, 83C520

87C530.MCU    DALLAS          87C530, 83C530

87C550.MCU    DALLAS          87C550

89C420.MCU    DALLAS          89C420

DS5000.MCU    DALLAS          5000FP, 5000, 5000T, 2250, 2250T

DS5001.MCU    DALLAS          5001FP, 5002FP, 5002FPM, 2251T, 2252T

MAX7651.MCU    Maxim           MAX7651, MAX7652

COM20051.MCU    SMC             COM20051

89C52.MCU    Atmel           89C52, 89C55, 89LV52, 89LV55, 87LV52,

80F52, 87F52

87F51RC.MCU    Atmel           87F51RC, 87F55, 87LV55

89C1051.MCU    Atmel           89C1051

89C2051.MCU    Atmel           89C2051, 89C4051, 89C1051U

89S8252.MCU    Atmel           89S8252, 89LS8252

89S51.MCU    Atmel           89S51

89S52.MCU    Atmel           89S52, 89LS52

89S53.MCU    Atmel           89S53, 89LS53

89S4D12.MCU    Atmel           89S4D12

73M2910.MCU    TDK             73M2910, 73M2910A

AN2131.MCU    Cypress         AN2121, AN2122, AN2125, AN2126,

AN2131, AN2135, AN2136

Appendix A

==========

ASEM-51 Error Messages

----------------------

A.1 Assembly Errors:

--------------------

Assembly errors apply to the consistency of the assembly language

program in syntax and semantics. If one of these errors is detected,

it is flagged in the list file, and program execution continues.

When assembly is finished, ASEM terminates with exit code 1:

Error Message                           Meaning

-----------------------------------------------------------------------------

address below segment base              Attempt to set the location counter

of the current segment below the

segment base address.

address out of range                    The address of a jump or call

instruction cannot be reached with

the selected addressing mode.

already a macro parameter               In a macro definition, a local symbol

is equal to a previously defined

parameter name.

argument exceeds end of line            A macro argument contains more

opening than closing angle brackets.

attempt to divide by zero               During evaluation of an assembly time

expression, the assembler has to

divide by zero.

binary operator expected                In this position of an expression,

only binary operators are allowed.

comma expected                          There should be a ',' character in

the marked position.

commands after END statement            The END statement is followed by

further assembler statements.

constant out of range                   A numerical constant is greater

than 65535.

duplicate local symbol                  In a macro definition, a local symbol

is defined multiple times or equal to

a previously defined parameter name.

duplicate parameter name                The parameter names of a macro are

not all different.

ENDIF statement expected                There are pending IFxx constructions,

which are not terminated with an

ENDIF meta instruction.

ENDM statement expected                 There are macro definitions, which

are not terminated with an ENDM

instruction.

expression out of range                 The result of an expression is too

big or too small for that purpose.

file name expected                      There should be a valid file name

in this position.

forward reference to macro              A macro has been called, before it

has been defined.

forward reference to register           A register type symbol has been used,

before it has been EQU'd or SET.

illegal character                       A statement contains characters, which

are not allowed in MCS-51 assembly

language.

illegal constant                        There are syntax errors in a

numeric constant.

illegal control statement               A statement is starting with an

unknown keyword beginning with a $.

illegal operand                         In this position of an expression,

a valid operand had been expected.

illegal statement syntax                A statement contains a syntax element,

which is not allowed in this context.

invalid base address                    A DATA address that is not bit-

addressable has been used on the

left side of a '.' operator.

invalid bit number                      A number greater than 7 has been

used on the right side of a '.'

operator.

invalid instruction                     The instruction has previously been

disabled with the $PHILIPS control.

macro type operand                      A macro type symbol is used as an

operand in a numeric expression.

maximum line length exceeded            During macro expansion, the replacement

of parameters and/or local symbols

increases the resulting line length

to more than 255 characters.

misplaced LOCAL instruction             In a macro definition, a LOCAL in-

struction is preceded by body lines.

misplaced macro instruction             A macro instruction is used outside

of a macro definition, or otherwise

misplaced in the program structure.

misplaced macro operator                A macro operator (<, >, !, %, &) has

been used in a wrong position.

module name already defined             There are more than one NAME

statements in the program.

must be known on first pass             The result of an expression must

fully evaluate on pass 1 of assembly.

must be preceded by $SAVE               A $RESTORE control occurs without

a preceding $SAVE control.

must be preceded by IFxx                An ELSEIFxx, ELSE or ENDIF meta

instruction occurs without a

preceding IFxx meta instruction.

no END statement found                  The program ends without an END

statement.

not allowed in BIT segment              Instruction is not allowed in a

BIT segment.

only allowed in BIT segment             Instruction is only allowed in a

BIT segment.

only allowed in CODE segment            Instruction is only allowed in a

CODE segment.

operand expected                        An instruction ends, before it is

syntactically complete.

phase error                             A symbol is evaluating to different

values on pass 1 and pass 2, or a

macro has been defined different on

pass 1 and pass 2.

Note:   This is a serious, internal assembler

-----   error, and should be reported to the

author immediately!

preceded by non-control lines           A primary control occurs after

statements that are no assembler

controls.

register type operand                   A register type symbol is used as an

operand in a numeric expression.

segment limit exceeded                  The location counter exceeds the

boundaries of the current segment.

segment type mismatch                   The segment type of an operand does

not match the type of the instruction.

string exceeds end of line              A character string is not properly

terminated with a quote.

symbol already defined                  Attempt to redefine a symbol, which

is already defined.

symbol name expected                    There should be a valid symbol name

in this position.

symbol not defined                      A symbol is referenced, which has

never been defined.

too many closing parentheses            An expression contains more closing

than opening parentheses.

too many opening parentheses            An expression contains more opening

than closing parentheses.

too many operands                       An instruction contains more operands

than expected.

unary operator expected                 In this position of an expression,

only unary operators are allowed.

user-defined error                      A user-defined error message has been

forced with the $ERROR control.

A.2 Runtime Errors:

-------------------

Runtime errors are operational errors, or I/O errors.

If one of these errors is detected, it is flagged on the console,

and ASEM is aborting with exit code 2:

Error Message                 Meaning

-----------------------------------------------------------------------------

access denied                 No privilege for attempted operation.

ambiguous option name         Not enough characters specified.

argument missing              Option requires an argument.

device or resource busy       Attempt to write to a busy device. (Linux)

disk full                     No more free disk space.

disk write-protected          Attempt to write to a write-protected disk.

drive not ready               Disk drive is off, or no media mounted.

duplicate file name           Attempt to overwrite an input or output file.

fatal I/O error               General (unknown) disk or device I/O error.

file not found                Source or include file not found. (DOS/Windows)

illegal option syntax         Option is not correctly specified.

invalid argument              Option has an illegal argument.

no input file                 There is no file name in the command line.

no such file or directory     Source or include file not found. (Linux)

not a directory               Path contains a non-directory name. (Linux)

out of memory                 Heap overflow!

path not found                Disk or directory not found. (DOS/Windows)

symbol is predefined          A /DEFINE option specifies a predefined symbol.

too many open files           No more free file handles.

too many parameters           More than three file names have been specified.

unknown option                Option is not implemented.

Only DOS command line options are mentioned above for simplification.

Under Linux, they have to be replaced by the corresponding Linux options.

Appendix B

==========

HEXBIN Error Messages

---------------------

B.1 Conversion Errors:

----------------------

Conversion errors apply to the consistency of Intel-HEX file and

program options. If one of these errors is detected, it is flagged

on the console, and HEXBIN is aborting with exit code 1:

Error Message                         Meaning

-----------------------------------------------------------------------------

checksum error                        Checksum is not correct.

data after EOF record                 Type 0 records after type 1 record.

file length out of range              /LENGTH option makes file too large.

fill-byte out of range                /FILL option defines byte value > 255.

hex file format error                 Certainly no Intel-HEX file.

illegal hex digit                     Character is no valid hex digit.

illegal record type                   Record type is none of 0 or 1.

invalid record length                 Record length doesn't match the record.

multiple EOF records                  More than one type 1 record.

no data records found                 File doesn't contain any type 0 records.

no EOF record found                   File ends without a type 1 record.

offset out of range                   /OFFSET option makes file too large.

record exceeds FFFFH                  Address space wrap around in record.

record exceeds file length            /LENGTH option made file too short.

Only DOS command line options are mentioned above for simplification.

Under Linux, they have to be replaced by the corresponding Linux options.

B.2 Runtime Errors:

-------------------

Runtime errors are operational errors, or I/O errors.

If one of these errors is detected, it is flagged on the console,

and HEXBIN is aborting with exit code 2:

Error Message                 Meaning

-----------------------------------------------------------------------------

access denied                 No privilege for attempted operation.

ambiguous option name         Not enough characters specified.

argument missing              Option requires an argument.

device or resource busy       Attempt to write to a busy device. (Linux)

disk full                     No more free disk space.

disk write-protected          Attempt to write to a write-protected disk.

drive not ready               Disk drive is off, or no media mounted.

duplicate file name           Attempt to overwrite an input or output file.

fatal I/O error               General (unknown) disk or device I/O error.

file not found                Intel-HEX file not found. (DOS/Windows)

illegal option syntax         Option is not correctly specified.

invalid argument              Option has an illegal argument.

no input file                 There is no file name in the command line.

no such file or directory     Intel-HEX file not found. (Linux)

not a directory               Path contains a non-directory name. (Linux)

path not found                Disk or directory not found. (DOS/Windows)

too many open files           No more free file handles.

too many parameters           More than two file names have been specified.

unknown option                Option is not implemented.

Appendix C

==========

Predefined Symbols

------------------

DATA Addresses:

---------------

P0          080H                        P1          090H

SP          081H                        SCON        098H

DPL         082H                        SBUF        099H

DPH         083H                        P2          0A0H

PCON        087H                        IE          0A8H

TCON        088H                        P3          0B0H

TMOD        089H                        IP          0B8H

TL0         08AH                        PSW         0D0H

TL1         08BH                        ACC         0E0H

TH0         08CH                        B           0F0H

TH1         08DH

BIT Addresses:

--------------

IT0         088H                        EA          0AFH

IE0         089H                        RXD         0B0H

IT1         08AH                        TXD         0B1H

IE1         08BH                        INT0        0B2H

TR0         08CH                        INT1        0B3H

TF0         08DH                        T0          0B4H

TR1         08EH                        T1          0B5H

TF1         08FH                        WR          0B6H

RI          098H                        RD          0B7H

TI          099H                        PX0         0B8H

RB8         09AH                        PT0         0B9H

TB8         09BH                        PX1         0BAH

REN         09CH                        PT1         0BBH

SM2         09DH                        PS          0BCH

SM1         09EH                        P           0D0H

SM0         09FH                        OV          0D2H

EX0         0A8H                        RS0         0D3H

ET0         0A9H                        RS1         0D4H

EX1         0AAH                        F0          0D5H

ET1         0ABH                        AC          0D6H

ES          0ACH                        CY          0D7H

CODE Addresses:

---------------

RESET      0000H                        EXTI1      0013H

EXTI0      0003H                        TIMER1     001BH

TIMER0     000BH                        SINT       0023H

Plain Numbers:

--------------

??ASEM_51  8051H                        ??VERSION  0130H

Appendix D

==========

Reserved Keywords

-----------------

Special Assembler Symbols:

--------------------------

$                                   location counter

A                                   accumulator

AB                                  A/B register pair

AR0,AR1,AR2,AR3,AR4,AR5,AR6,AR7     direct register addresses

C                                   carry flag

DPTR                                data pointer

PC                                  program counter

R0, R1, R2, R3, R4, R5, R6, R7      registers

Instruction Mnemonics

---------------------

ACALL       DA          JNB         MUL         RR

ADD         DEC         JNC         NOP         RRC

ADDC        DIV         JNZ         ORL         SETB

AJMP        DJNZ        JZ          POP         SJMP

ANL         INC         LCALL       PUSH        SUBB

CALL        JB          LJMP        RET         SWAP

CJNE        JBC         MOV         RETI        XCH

CLR         JC          MOVC        RL          XCHD

CPL         JMP         MOVX        RLC         XRL

Pseudo Instructions

-------------------

AT          DATA        DSEG        IDATA       SET

BIT         DB          DW          ISEG        USING

BSEG        DBIT        END         NAME        XDATA

CODE        DS          EQU         ORG         XSEG

CSEG

Operators

---------

AND         GT          LOW         NE          SHL

EQ          HIGH        LT          NOT         SHR

GE          LE          MOD         OR          XOR

Assembler Controls

------------------

$COND       $GO         $NODEBUG    $NOSYMBOLS  $RS

$CONDONLY   $IC         $NOGE       $NOTABS     $SA

$DA         $INCLUDE    $NOGEN      $NOXR       $SAVE

$DATE       $LI         $NOLI       $NOXREF     $SB

$DB         $LIST       $NOLIST     $PAGELENGTH $SYMBOLS

$DEBUG      $MACRO      $NOMACRO    $PAGEWIDTH  $TITLE

$EJ         $MO         $NOMO       $PAGING     $TT

$EJECT      $MOD51      $NOMOD51    $PHILIPS    $WARNING

$ERROR      $MR         $NOMR       $PI         $XR

$GE         $NOBUILTIN  $NOPAGING   $PL         $XREF

$GEN        $NOCOND     $NOPI       $PW

$GENONLY    $NODB       $NOSB       $RESTORE

Meta Instructions

-----------------

ELSE        ELSEIFN     ENDM        IFDEF       LOCAL

ELSEIF      ELSEIFNB    EXITM       IFN         MACRO

ELSEIFB     ELSEIFNDEF  IF          IFNB        REPT

ELSEIFDEF   ENDIF       IFB         IFNDEF

Appendix E

==========

Specification of the Intel-HEX Format

-------------------------------------

This object file format is supported by many cross assemblers, utilities,

and most EPROM programmers.

An Intel-HEX file is a 7-bit ASCII text file, that contains a sequence of

data records and an end record. Every record is a line of text that starts

with a colon and ends with CR and LF.

Data records contain up to 16 data bytes, a 16-bit load address, a record

type byte and an 8-bit checksum. All numbers are represented by upper case

ASCII-hex characters.

DATA RECORD:

Byte 1               colon (:)

2 and 3         number of binary data bytes for this record

4 and 5         load address for this record, high byte

6 and 7         load address  "    "     "     low byte

8 and 9         record type: 00  (data record)

10 to  x         data bytes, two characters each

x+1 to  x+2       checksum (two characters)

x+3 to  x+4       CR and LF

A typical data record looks like

:10E0000002E003E4F588758910F58DF58BD28E302A

The end record is the last line of the file.

In principle it is structured like a data record, but the number of data

bytes is 00, the record type is 01 and the load-address field is 0000.

END RECORD:

Byte 1               colon (:)

2 and 3         00   (number of data bytes)

4 and 5         00   (load address, high byte)

6 and 7         00   (load address, low byte)

8 and 9         record type: 01  (end record)

10 and 11        checksum (two characters)

12 and 13        CR and LF

The typical END record looks like

:00000001FF

The checksum is the two's complement of the 8-bit sum, without carry, of

the byte count, the two load address bytes, the record type byte and all

data bytes.

Appendix F

==========

The ASCII Character Set

-----------------------

hex |   00   10   20   30   40   50   60   70

-----+------------------------------------------

0  |  NUL  DLE         0    @    P    `    p

1  |  SOH  DC1    !    1    A    Q    a    q

2  |  STX  DC2    "    2    B    R    b    r

3  |  ETX  DC3    #    3    C    S    c    s

4  |  EOT  DC4    $    4    D    T    d    t

5  |  ENQ  NAK    %    5    E    U    e    u

6  |  ACK  SYN    &    6    F    V    f    v

7  |  BEL  ETB    '    7    G    W    g    w

8  |   BS  CAN    (    8    H    X    h    x

9  |   HT   EM    )    9    I    Y    i    y

A  |   LF  SUB    *    :    J    Z    j    z

B  |   VT  ESC    +    ;    K    [    k    {

C  |   FF   FS    ,    <    L    \    l    |

D  |   CR   GS    -    =    M    ]    m    }

E  |   SO   RS    .    >    N    ^    n    ~

F  |   SI   US    /    ?    O    _    o  DEL

Appendix G

==========

Literature

----------

Order Number

Intel:   MCS(R) 51 Microcontroller Family User's Manual

MCS-51 Macro Assembler User's Guide

8-Bit Embedded Controllers 1990

8XC51RA/RB/RC CHMOS Single-Chip 8-Bit Microcontroller     272659-002

8XC51RA/RB/RC Hardware Description, February 1995         272668-001

83C51KB High Performance Keyboard Microcontroller         272800-001

83C51KB Hardware Description                              272801-001

MCS 51 Microcontroller Family User's Manual, Feb 1994     272383-002

8XC151SA and 8XC151SB Hardware Description, June 1996     272832-001

Embedded Microcontrollers, 1997                           270646-009

8x931AA, 8x931HA USB Peripheral Controller User's Manual  Sept. 97

SIEMENS: SAB 80512/80532 User's Manual                      B2-B3808-X-X-7600

SAB 80515/80535 User's Manual                      B2-B3976-X-X-7600

SAB 80C515/80C535 Data Sheet

SAB 80C515A/83C515A-5 Addendum                   B158-H6613-X-X-7600

SAB 80C515A/83C515A-5 Data Sheet                 B158-H6605-X-X-7600

SAB 80C517/80C537 User's Manual                  B258-B6075-X-X-7600

SAB 80C517A/83C517A-5 Addendum                   B158-H6612-X-X-7600

SAB 80C517A/83C517A-5 Data Sheet                 B158-H6581-X-X-7600

SIEMENS Microcontrollers Data Catalog            B158-H6569-X-X-7600

SAB 80513/8352-5 Data Sheet                      B158-B6245-X-X-7600

SAB-C501 User's Manual                           B158-H6723-G1-X-7600

SAB-C502 User's Manual                           B158-H6722-G1-X-7600

SAB-C503 User's Manual                           B158-H6650-G1-X-7600

Application Notes and User Manuals, CD-ROM       B193-H6900-X-X-7400

C504 8-Bit CMOS Microcontroller User's Manual    B158-H6958-X-X-7600

C509-L 8-Bit CMOS Microcontroller User's Manual  B158-H6973-X-X-7600

C515C 8-Bit CMOS Microcontroller User's Manual   B158-H6981-X-X-7600

Infineon: C515 8-Bit CMOS Microcontroller User's Manual                 04.98

C515A 8-Bit CMOS Microcontroller User's Manual                08.97

C517A 8-Bit CMOS Microcontroller User's Manual                01.99

C540U/C541U 8-Bit CMOS Microcontroller Data Sheet             10.97

C513AO 8-bit CMOS Microcontroller User's Manual               05.99

C505/C505C 8-Bit CMOS Microcontroller User's Manual           08.97

C505A/C505CA 8-Bit CMOS Microcontroller, Addendum             09.97

C505L 8-bit CMOS Microcontroller, Data Sheet                  06.99

C508 8-Bit CMOS Microcontroller User's Manual              May 2001

C868 8-Bit CMOS Microcontroller User's Manual, V 0.5.1     Jan 2002

Philips: PCB83C552 User's Manual

PCB83C552, PCB80C552 Development Data

8051-Based 8-Bit Microcontrollers - Data Handbook 1994

83C754/87C754 Preliminary Specification, 1998 Apr 23

87LPC762 Data Sheet, 2001 Oct 26

87LPC768 Data Sheet, 2002 Mar 12

P80C3xX2, P80C5xX2, P87C5xX2 Data Sheet, 2002 Jun 12

P89C51X2/52X2/54X2/58X2 Data Sheet, 2002 Jun 05

AMD:     Eight-Bit 80C51 Embedded Processors - Data Book 1990

OKI:     MSM80C154, MSM83C154 User's Manual

Microcontroller Data Book, 5th Edition 1990

TDK:     TSC 73M2910/2910A Microcontroller, 10/22/96 - rev.

DALLAS:  High-Speed Micro User's Guide, V1.3 January 1994, 011994

Secure Microcontroller User's Guide, 062001

DS80C310 High-Speed Micro, 090198

DS80C320/DS80C323 High-Speed/Low-Power Micro, 070196

DS80C390 Dual CAN High-Speed Microprocessor, 090799

DS87C520/DS83C520 EPROM/ROM High-Speed Micro, 110195

DS87C530/DS83C530 EPROM/ROM Micro with Real-Time Clock, 112299

DS87C550 EPROM High-Speed Micro with A/D and PWM, 091698

DS89C420 Ultra High-Speed Microcontroller User's Guide, 020602

Maxim:   MAX7651/MAX7652 Programmer's Reference Manual

MAX7651/MAX7652 Data Sheet, 19-2119; Rev 0; 8/01

Atmel:   AT89C51, 8-Bit Microcontroller with 4 Kbytes Flash             0265E

AT89C52, 8-Bit Microcontroller with 8 Kbytes Flash             0313E

AT89C55, 8 bit Microcontroller with 20K bytes Flash

AT89LV51, 8-Bit Microcontroller with 4 Kbytes Flash            0303C

AT89LV52, 8-Bit Microcontroller with 8 Kbytes Flash

AT89C1051, 8-Bit Microcontroller with 1 Kbyte Flash            0366C

AT89C2051, 8-Bit Microcontroller with 2 Kbytes Flash           0368C

AT89C4051, 8-Bit Microcontroller with 4K Bytes Flash     Preliminary

AT89C1051U, 8-Bit Microcontroller with 1K Bytes Flash    Preliminary

AT89S8252, 8 Bit Microcontroller with 8K bytes Flash     Preliminary

AT89LS8252, 8-Bit Microcontroller with 8K Bytes Flash    0850B-B-12/97

AT89S51, 8-bit Microcontroller with 4K Bytes ISP Flash   2487A-10/01

AT89S52, 8-bit Microcontroller with 8K Bytes ISP Flash   1919A-07/01

AT89LS52, 8-bit LV Microcontroller with 8K Bytes ISP Flash 2601A-12/01

AT89S53, 8-Bit Microcontroller with 12K Bytes Flash      Preliminary

AT89LS53, 8-Bit Microcontroller with 12K Bytes Flash     0851B-B-12/97

AT89LV55, 8-Bit Microcontroller with 20K bytes Flash     0811A-A-7/97

AT80F51, 8-Bit Microcontroller with 4K Bytes QuickFlash  0979A-A-12/97

AT87F51, 8-Bit Microcontroller with 4K Bytes QuickFlash  1012A-02/98

AT80F52, 8-Bit Microcontroller with 8K Bytes QuickFlash  0980A-A-12/97

AT87F52, 8-Bit Microcontroller with 8K Bytes QuickFlash  1011A-02/98

AT89S4D12, 8-Bit Microcontroller with 132K Bytes Flash   0921A-A-12/97

AT87F51RC, 8-Bit Microcontroller with 32K Bytes QuickFlash 1106B-12/98

AT87F55, 8-Bit Microcontroller with 20K Bytes QuickFlash 1147A-05/99

AT87LV51, 8-bit Microcontroller with 4K Bytes QuickFlash 1602A-04/00

AT87LV52, 8-Bit Microcontroller with 8K Bytes QuickFlash 1437A-07/99

AT87LV55, 8-bit Microcontroller with 20K Bytes QuickFlash 1609A-04/00

Cypress: The EZ-USB Integrated Circuit, Technical Reference Manual Version 1.9

German Literature:

------------------

Andreas Roth:  Das Microcontroller Kochbuch MCS51,

6. Aufl. 2002, mitp-Verlag, ISBN 3-8266-0722-8

Appendix H

==========

Trademarks

----------

ASEM-51 is a trademark of W.W. Heinz.

MCS-51 and ASM51 are trademarks of Intel Corporation.

Turbo-Pascal and Borland-Pascal are trademarks of Borland International, Inc.

Delphi is a trademark of Borland International, Inc.

Turbo C++ and Borland C++ are trademarks of Borland International, Inc.

Turbo-Assembler is a trademark of Borland International, Inc.

IBM-PC, IBM-XT, IBM-AT and OS/2 are trademarks of IBM Corporation.

MS-DOS and Windows are trademarks of Microsoft Corporation.

Novell DOS is a trademark of Novell, Inc.

BRIEF is a trademark of SDC Partners II L.P.

4DOS is a registered trademark of JP Software Inc.

Linux is a trademark of Linus Torvalds.

FreePascal is a trademark of Florian Klaempfl.

All device codes of 8051 derivatives are trademarks of the manufacturers.

Other brand and product names are trademarks of their respective holders.

Appendix I

==========

8051 Instructions in numerical Order

------------------------------------

Abbreviations:      direct   =   8-bit DATA address in internal memory

const8   =   8-bit constant in CODE memory

const16  =  16-bit constant in CODE memory

addr16   =  16-bit long CODE address

addr11   =  11-bit absolute CODE address

rel      =  signed 8-bit relative CODE address

bit      =   8-bit BIT address in internal memory

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

00      NOP                           1                    1

01      AJMP    addr11                2                    2

02      LJMP    addr16                3                    2

03      RR      A                     1                    1

04      INC     A                     1              P     1

05      INC     direct                2                    1

06      INC     @R0                   1                    1

07      INC     @R1                   1                    1

08      INC     R0                    1                    1

09      INC     R1                    1                    1

0A      INC     R2                    1                    1

0B      INC     R3                    1                    1

0C      INC     R4                    1                    1

0D      INC     R5                    1                    1

0E      INC     R6                    1                    1

0F      INC     R7                    1                    1

10      JBC     bit, rel              3                    2

11      ACALL   addr11                2                    2

12      LCALL   addr16                3                    2

13      RRC     A                     1     CY       P     1

14      DEC     A                     1              P     1

15      DEC     direct                2                    1

16      DEC     @R0                   1                    1

17      DEC     @R1                   1                    1

18      DEC     R0                    1                    1

19      DEC     R1                    1                    1

1A      DEC     R2                    1                    1

1B      DEC     R3                    1                    1

1C      DEC     R4                    1                    1

1D      DEC     R5                    1                    1

1E      DEC     R6                    1                    1

1F      DEC     R7                    1                    1

20      JB      bit, rel              3                    2

21      AJMP    addr11                2                    2

22      RET                           1                    2

23      RL      A                     1                    1

24      ADD     A, #const8            2     CY AC OV P     1

25      ADD     A, direct             2     CY AC OV P     1

26      ADD     A, @R0                1     CY AC OV P     1

27      ADD     A, @R1                1     CY AC OV P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

28      ADD     A, R0                 1     CY AC OV P     1

29      ADD     A, R1                 1     CY AC OV P     1

2A      ADD     A, R2                 1     CY AC OV P     1

2B      ADD     A, R3                 1     CY AC OV P     1

2C      ADD     A, R4                 1     CY AC OV P     1

2D      ADD     A, R5                 1     CY AC OV P     1

2E      ADD     A, R6                 1     CY AC OV P     1

2F      ADD     A, R7                 1     CY AC OV P     1

30      JNB     bit, rel              3                    2

31      ACALL   addr11                2                    2

32      RETI                          1                    2

33      RLC     A                     1     CY       P     1

34      ADDC    A, #const8            2     CY AC OV P     1

35      ADDC    A, direct             2     CY AC OV P     1

36      ADDC    A, @R0                1     CY AC OV P     1

37      ADDC    A, @R1                1     CY AC OV P     1

38      ADDC    A, R0                 1     CY AC OV P     1

39      ADDC    A, R1                 1     CY AC OV P     1

3A      ADDC    A, R2                 1     CY AC OV P     1

3B      ADDC    A, R3                 1     CY AC OV P     1

3C      ADDC    A, R4                 1     CY AC OV P     1

3D      ADDC    A, R5                 1     CY AC OV P     1

3E      ADDC    A, R6                 1     CY AC OV P     1

3F      ADDC    A, R7                 1     CY AC OV P     1

40      JC      rel                   2                    2

41      AJMP    addr11                2                    2

42      ORL     direct, A             2                    1

43      ORL     direct, #const8       3                    2

44      ORL     A, #const8            2              P     1

45      ORL     A, direct             2              P     1

46      ORL     A, @R0                1              P     1

47      ORL     A, @R1                1              P     1

48      ORL     A, R0                 1              P     1

49      ORL     A, R1                 1              P     1

4A      ORL     A, R2                 1              P     1

4B      ORL     A, R3                 1              P     1

4C      ORL     A, R4                 1              P     1

4D      ORL     A, R5                 1              P     1

4E      ORL     A, R6                 1              P     1

4F      ORL     A, R7                 1              P     1

50      JNC     rel                   2                    2

51      ACALL   addr11                2                    2

52      ANL     direct, A             2                    1

53      ANL     direct, #const8       3                    2

54      ANL     A, #const8            2              P     1

55      ANL     A, direct             2              P     1

56      ANL     A, @R0                1              P     1

57      ANL     A, @R1                1              P     1

58      ANL     A, R0                 1              P     1

59      ANL     A, R1                 1              P     1

5A      ANL     A, R2                 1              P     1

5B      ANL     A, R3                 1              P     1

5C      ANL     A, R4                 1              P     1

5D      ANL     A, R5                 1              P     1

5E      ANL     A, R6                 1              P     1

5F      ANL     A, R7                 1              P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

60      JZ      rel                   2                    2

61      AJMP    addr11                2                    2

62      XRL     direct, A             2                    1

63      XRL     direct, #const8       3                    2

64      XRL     A, #const8            2              P     1

65      XRL     A, direct             2              P     1

66      XRL     A, @R0                1              P     1

67      XRL     A, @R1                1              P     1

68      XRL     A, R0                 1              P     1

69      XRL     A, R1                 1              P     1

6A      XRL     A, R2                 1              P     1

6B      XRL     A, R3                 1              P     1

6C      XRL     A, R4                 1              P     1

6D      XRL     A, R5                 1              P     1

6E      XRL     A, R6                 1              P     1

6F      XRL     A, R7                 1              P     1

70      JNZ     rel                   2                    2

71      ACALL   addr11                2                    2

72      ORL     C, bit                2     CY             2

73      JMP     @A+DPTR               1                    2

74      MOV     A, #const8            2              P     1

75      MOV     direct, #const8       3                    2

76      MOV     @R0, #const8          2                    1

77      MOV     @R1, #const8          2                    1

78      MOV     R0, #const8           2                    1

79      MOV     R1, #const8           2                    1

7A      MOV     R2, #const8           2                    1

7B      MOV     R3, #const8           2                    1

7C      MOV     R4, #const8           2                    1

7D      MOV     R5, #const8           2                    1

7E      MOV     R6, #const8           2                    1

7F      MOV     R7, #const8           2                    1

80      SJMP    rel                   2                    2

81      AJMP    addr11                2                    2

82      ANL     C, bit                2     CY             2

83      MOVC    A, @A+PC              1              P     2

84      DIV     AB                    1     CY    OV P     4

85      MOV     direct, direct        3                    2

86      MOV     direct, @R0           2                    2

87      MOV     direct, @R1           2                    2

88      MOV     direct, R0            2                    2

89      MOV     direct, R1            2                    2

8A      MOV     direct, R2            2                    2

8B      MOV     direct, R3            2                    2

8C      MOV     direct, R4            2                    2

8D      MOV     direct, R5            2                    2

8E      MOV     direct, R6            2                    2

8F      MOV     direct, R7            2                    2

90      MOV     DPTR, #const16        3                    2

91      ACALL   addr11                2                    2

92      MOV     bit, C                2                    2

93      MOVC    A, @A+DPTR            1              P     2

94      SUBB    A, #const8            2     CY AC OV P     1

95      SUBB    A, direct             2     CY AC OV P     1

96      SUBB    A, @R0                1     CY AC OV P     1

97      SUBB    A, @R1                1     CY AC OV P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

98      SUBB    A, R0                 1     CY AC OV P     1

99      SUBB    A, R1                 1     CY AC OV P     1

9A      SUBB    A, R2                 1     CY AC OV P     1

9B      SUBB    A, R3                 1     CY AC OV P     1

9C      SUBB    A, R4                 1     CY AC OV P     1

9D      SUBB    A, R5                 1     CY AC OV P     1

9E      SUBB    A, R6                 1     CY AC OV P     1

9F      SUBB    A, R7                 1     CY AC OV P     1

A0      ORL     C, /bit               2     CY             2

A1      AJMP    addr11                2                    2

A2      MOV     C, bit                2     CY             1

A3      INC     DPTR                  1                    2

A4      MUL     AB                    1     CY    OV P     4

A5      illegal opcode

A6      MOV     @R0, direct           2                    2

A7      MOV     @R1, direct           2                    2

A8      MOV     R0, direct            2                    2

A9      MOV     R1, direct            2                    2

AA      MOV     R2, direct            2                    2

AB      MOV     R3, direct            2                    2

AC      MOV     R4, direct            2                    2

AD      MOV     R5, direct            2                    2

AE      MOV     R6, direct            2                    2

AF      MOV     R7, direct            2                    2

B0      ANL     C, /bit               2     CY             2

B1      ACALL   addr11                2                    2

B2      CPL     bit                   2                    1

B3      CPL     C                     1     CY             1

B4      CJNE    A, #const8, rel       3     CY             2

B5      CJNE    A, direct, rel        3     CY             2

B6      CJNE    @R0, #const8, rel     3     CY             2

B7      CJNE    @R1, #const8, rel     3     CY             2

B8      CJNE    R0, #const8, rel      3     CY             2

B9      CJNE    R1, #const8, rel      3     CY             2

BA      CJNE    R2, #const8, rel      3     CY             2

BB      CJNE    R3, #const8, rel      3     CY             2

BC      CJNE    R4, #const8, rel      3     CY             2

BD      CJNE    R5, #const8, rel      3     CY             2

BE      CJNE    R6, #const8, rel      3     CY             2

BF      CJNE    R7, #const8, rel      3     CY             2

C0      PUSH    direct                2                    2

C1      AJMP    addr11                2                    2

C2      CLR     bit                   2                    1

C3      CLR     C                     1     CY             1

C4      SWAP    A                     1                    1

C5      XCH     A, direct             2              P     1

C6      XCH     A, @R0                1              P     1

C7      XCH     A, @R1                1              P     1

C8      XCH     A, R0                 1              P     1

C9      XCH     A, R1                 1              P     1

CA      XCH     A, R2                 1              P     1

CB      XCH     A, R3                 1              P     1

CC      XCH     A, R4                 1              P     1

CD      XCH     A, R5                 1              P     1

CE      XCH     A, R6                 1              P     1

CF      XCH     A, R7                 1              P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

D0      POP     direct                2                    2

D1      ACALL   addr11                2                    2

D2      SETB    bit                   2                    1

D3      SETB    C                     1     CY             1

D4      DA      A                     1     CY       P     1

D5      DJNZ    direct, rel           3                    2

D6      XCHD    A, @R0                1              P     1

D7      XCHD    A, @R1                1              P     1

D8      DJNZ    R0, rel               2                    2

D9      DJNZ    R1, rel               2                    2

DA      DJNZ    R2, rel               2                    2

DB      DJNZ    R3, rel               2                    2

DC      DJNZ    R4, rel               2                    2

DD      DJNZ    R5, rel               2                    2

DE      DJNZ    R6, rel               2                    2

DF      DJNZ    R7, rel               2                    2

E0      MOVX    A, @DPTR              1              P     2

E1      AJMP    addr11                2                    2

E2      MOVX    A, @R0                1              P     2

E3      MOVX    A, @R1                1              P     2

E4      CLR     A                     1              P     1

E5      MOV     A, direct             2              P     1

E6      MOV     A, @R0                1              P     1

E7      MOV     A, @R1                1              P     1

E8      MOV     A, R0                 1              P     1

E9      MOV     A, R1                 1              P     1

EA      MOV     A, R2                 1              P     1

EB      MOV     A, R3                 1              P     1

EC      MOV     A, R4                 1              P     1

ED      MOV     A, R5                 1              P     1

EE      MOV     A, R6                 1              P     1

EF      MOV     A, R7                 1              P     1

F0      MOVX    @DPTR, A              1                    2

F1      ACALL   addr11                2                    2

F2      MOVX    @R0, A                1                    2

F3      MOVX    @R1, A                1                    2

F4      CPL     A                     1              P     1

F5      MOV     direct, A             2                    1

F6      MOV     @R0, A                1                    1

F7      MOV     @R1, A                1                    1

F8      MOV     R0, A                 1                    1

F9      MOV     R1, A                 1                    1

FA      MOV     R2, A                 1                    1

FB      MOV     R3, A                 1                    1

FC      MOV     R4, A                 1                    1

FD      MOV     R5, A                 1                    1

FE      MOV     R6, A                 1                    1

FF      MOV     R7, A                 1                    1

Appendix J

==========

8051 Instructions in lexical Order

----------------------------------

Abbreviations:      direct   =   8-bit DATA address in internal memory

const8   =   8-bit constant in CODE memory

const16  =  16-bit constant in CODE memory

addr16   =  16-bit long CODE address

addr11   =  11-bit absolute CODE address

rel      =  signed 8-bit relative CODE address

bit      =   8-bit BIT address in internal memory

i  =  register numbers 0 or 1

n  =  register numbers 0 thru 7

a  =  32 * m

m  =  the 3 most significant bits of an absolute address

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

11+a    ACALL   addr11                2                    2

24      ADD     A, #const8            2     CY AC OV P     1

26+i    ADD     A, @Ri                1     CY AC OV P     1

25      ADD     A, direct             2     CY AC OV P     1

28+n    ADD     A, Rn                 1     CY AC OV P     1

34      ADDC    A, #const8            2     CY AC OV P     1

36+i    ADDC    A, @Ri                1     CY AC OV P     1

35      ADDC    A, direct             2     CY AC OV P     1

38+n    ADDC    A, Rn                 1     CY AC OV P     1

01+a    AJMP    addr11                2                    2

54      ANL     A, #const8            2              P     1

56+i    ANL     A, @Ri                1              P     1

55      ANL     A, direct             2              P     1

58+n    ANL     A, Rn                 1              P     1

B0      ANL     C, /bit               2     CY             2

82      ANL     C, bit                2     CY             2

53      ANL     direct, #const8       3                    2

52      ANL     direct, A             2                    1

B6+i    CJNE    @Ri, #const8, rel     3     CY             2

B4      CJNE    A, #const8, rel       3     CY             2

B5      CJNE    A, direct, rel        3     CY             2

B8+n    CJNE    Rn, #const8, rel      3     CY             2

E4      CLR     A                     1              P     1

C2      CLR     bit                   2                    1

C3      CLR     C                     1     CY             1

F4      CPL     A                     1              P     1

B2      CPL     bit                   2                    1

B3      CPL     C                     1     CY             1

D4      DA      A                     1     CY       P     1

16+i    DEC     @Ri                   1                    1

14      DEC     A                     1              P     1

15      DEC     direct                2                    1

18+n    DEC     Rn                    1                    1

84      DIV     AB                    1     CY    OV P     4

D5      DJNZ    direct, rel           3                    2

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

D8+n    DJNZ    Rn, rel               2                    2

06+i    INC     @Ri                   1                    1

04      INC     A                     1              P     1

05      INC     direct                2                    1

A3      INC     DPTR                  1                    2

08+n    INC     Rn                    1                    1

20      JB      bit, rel              3                    2

10      JBC     bit, rel              3                    2

40      JC      rel                   2                    2

73      JMP     @A+DPTR               1                    2

30      JNB     bit, rel              3                    2

50      JNC     rel                   2                    2

70      JNZ     rel                   2                    2

60      JZ      rel                   2                    2

12      LCALL   addr16                3                    2

02      LJMP    addr16                3                    2

76+i    MOV     @Ri, #const8          2                    1

F6+i    MOV     @Ri, A                1                    1

A6+i    MOV     @Ri, direct           2                    2

74      MOV     A, #const8            2              P     1

E6+i    MOV     A, @Ri                1              P     1

E5      MOV     A, direct             2              P     1

E8+n    MOV     A, Rn                 1              P     1

92      MOV     bit, C                2                    2

A2      MOV     C, bit                2     CY             1

75      MOV     direct, #const8       3                    2

86+i    MOV     direct, @Ri           2                    2

F5      MOV     direct, A             2                    1

85      MOV     direct, direct        3                    2

88+n    MOV     direct, Rn            2                    2

90      MOV     DPTR, #const16        3                    2

78+n    MOV     Rn, #const8           2                    1

F8+n    MOV     Rn, A                 1                    1

A8+n    MOV     Rn, direct            2                    2

93      MOVC    A, @A+DPTR            1              P     2

83      MOVC    A, @A+PC              1              P     2

F0      MOVX    @DPTR, A              1                    2

F2+i    MOVX    @Ri, A                1                    2

E0      MOVX    A, @DPTR              1              P     2

E2+i    MOVX    A, @Ri                1              P     2

A4      MUL     AB                    1     CY    OV P     4

00      NOP                           1                    1

44      ORL     A, #const8            2              P     1

46+i    ORL     A, @Ri                1              P     1

45      ORL     A, direct             2              P     1

48+n    ORL     A, Rn                 1              P     1

A0      ORL     C, /bit               2     CY             2

72      ORL     C, bit                2     CY             2

43      ORL     direct, #const8       3                    2

42      ORL     direct, A             2                    1

D0      POP     direct                2                    2

C0      PUSH    direct                2                    2

22      RET                           1                    2

32      RETI                          1                    2

23      RL      A                     1                    1

33      RLC     A                     1     CY       P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

03      RR      A                     1                    1

13      RRC     A                     1     CY       P     1

D2      SETB    bit                   2                    1

D3      SETB    C                     1     CY             1

80      SJMP    rel                   2                    2

94      SUBB    A, #const8            2     CY AC OV P     1

96+i    SUBB    A, @Ri                1     CY AC OV P     1

95      SUBB    A, direct             2     CY AC OV P     1

98+n    SUBB    A, Rn                 1     CY AC OV P     1

C4      SWAP    A                     1                    1

C6+i    XCH     A, @Ri                1              P     1

C5      XCH     A, direct             2              P     1

C8+n    XCH     A, Rn                 1              P     1

D6+i    XCHD    A, @Ri                1              P     1

64      XRL     A, #const8            2              P     1

66+i    XRL     A, @Ri                1              P     1

65      XRL     A, direct             2              P     1

68+n    XRL     A, Rn                 1              P     1

63      XRL     direct, #const8       3                    2

62      XRL     direct, A             2                    1